A modeling study of impact of emission control strategies on haze in Zhongshan, China using WRF-CMAQ

Jianhua Mai1,2, Tao Deng2, Lingling Yu3, Xuejiao Deng2, Haobo Tan2, Shiqiang Wang4, Xiantong Liu2

1 Zhongshan Meteorological Service, Zhongshan 528400, China

2 Institute of Tropical and Marine Meteorology/Guangdong Provincial Key Laboratory of Regional Numerical Weather Prediction, Guangzhou, China Meteorological Administration, Guangzhou 510080, China

3 Guangdong Meteorological Observatory, Guangzhou 510080, China

4 Zhuhai Meteorological Service, Zhuhai 519000, China

Corresponding Author: Tao Deng

Institution: Institute of Tropical and Marine Meteorology/Guangdong Provincial Key Laboratory of Regional Numerical Weather Prediction, Guangzhou, China

Address: 6 Fujin Road, Yuexiu, Guangzhou, China

Tel: +86 13560376450

E-mail: tdeng@grmc.gov.cn

Abstract: A WRF-CMAQ modeling system is used to study the impact of emission control strategies and weather conditions on haze pollution in Zhongshan, Guangdong Province, China. One full month simulations for January 2014 are completed and evaluated with the surface observational data. The results show that the modeling system is able to reproduce the meteorological parameters and PM2.5 concentrations. Several sensitivity studies are completed to quantify the relative percentage contributions of local emissions versus regional emissions to the PM2.5 concentrations under different weather conditions. The results indicate that the local emissions from Zhongshan play higher contribution than the emissions from outside (or regional transport) when no cold fronts appear in the Pearl River Delta (PRD) region (i.e., 58% contribution from local emission versus 42% contribution from the regional transport). The contribution of the emissions from the regional transport is increased to 76% when a strong cold front appears. Furthermore, the sensitivity study demonstrates that the emission reduction has accumulative effect and this effect is varied with weather conditions. In specific, PM2.5concentrations on the first, second and third day are reduced by 47%, 52%, and 58%, respectively, after the local emissions are turned off when there is no cold front appears. Finally, a case study shows that industrial, residential, and mobile sources account for 24%, 22%, and 15% of PM2.5, respectively during a heavy haze pollution event in Zhongshan.,

Keywords: Emission control, Haze, PM2.5, WRF-CMAQ

1. Introduction

With rapid development of urbanization and industrialization, anthropogenic emissions have been increased significantly over the past several decades [1]. Haze has become a very serious air pollution problem in the Pearl River Delta region (PRD) and other regions in China [2]. High ambient levels of particulate matters with diameters less than 2.5 micrometer (PM2.5) can reduce visibility significantly, attenuate solar radiation, and pose harmful impact on human health [3-4].The observational data show that the number of haze episode days (the episode day is defined when daily mean PM2.5 concentration exceeds 75µg·m-3, the National Ambient Air Quality Standard for PM2.5 in China) was over 100 in the PRD over the past decade [5]. The haze pollution has received a great concern from the governments to the general public. Thus, understanding the relative contribution of local emissions and regional transport is extremely important to reduce PM2.5 concentrations.

Many research efforts have been devoted to the chemical composition characteristics of PM2.5, impact of weather systems and other meteorological fields on haze over the past several decades in the PRD region [6-13]. The monitoring data show that carbonaceous and inorganic secondary aerosols are the two major contributors to PM2.5 in the PRD [14]. Also some early study has already pointed out that emissions outside of the PRD contribute a significant fraction of overall PM2.5 in the PRD, but they did not quantify the contributions [15]. With the rapid development of computer technologies, numerical air quality models have become important tools for air pollution studies [16]. At the beginning, the Hybrid Single Particle Lagrangian Integrated Trajectory Model (Hysplit) was used to track the sources of air masses that have important impact on haze pollution events in the PRD region [17-18]. Later, some more complicated numerical models were utilized to investigate the air pollution events [19-20]. The Community Multi-scale Air Quality (CMAQ) model has been widely used to study pollution events in PRD [21-25]. These studies have showed that haze pollution severity in PRD is mainly controlled by anthropogenic emissions with strong spatial variation. They also pointed out that weather conditions are the key to trigger the haze pollution events [26]. However, very few studies have identified the relative contribution of local emissions and regional transport to PM2.5 concentrations in the PRD region. Thus, there is an urgent need in accurate quantification of relative contributions of local emissions and regional transport to PM2.5for formulating more effective emission control strategies in the PRD.

Zhongshan, located in the middle part of the PRD, suffers from the same haze pollutions as other cities in recent years. However, both observational and modeling studies of PM2.5 monitoring and efficient control strategy are very lacking [27]. This study is aimed at providing some scientific evidences for local policy-makers to develop more effective emission controls. For this purpose, the offline coupling system, Weather Research and Forecast (WRF) and CMAQ, is used to simulate the haze pollution and to study the impact of local emission and regional transport to PM2.5 during haze events in Zhongshan during January 2014, a month 16 haze episode days. In addition, the contributions from different anthropogenic emissions will be quantified

2. Methods

2.1 Descriptions of models and simulation configurations

The WRF model (version 3.3.1) [28] is used to provide meteorological inputs to drive CMAQ. 6-hourly Final Operational Global Analysis data with 1°×1°resolution from the National Center for Environmental Prediction (NCEP) are used for initial and boundary conditions for the WRF simulations. The CMAQ model (version 5.0.2) is used to simulate different physical and chemical processes of gaseous pollutants and particulate matter (PM) compositions. The carbon bond (CB05) gas-phase chemical mechanism [29] and the AERO5 module [30] are utilized in the simulations.

b)

a)

Figure 1. a) Two-nested modeling domains for WRF (black solid boxes) and CMAQ ( blue dashed boxes) models; b) different parts of emissions in different colors (details will be given in Section2.2) (Cities of PRD in the right picture: Zhongshan (ZS), Guangzhou (GZ), Foshan (FS), Jiangmen (JM), Zhuhai (ZH), Dongguan (DG) and Shenzhen (SZ)).

Figure 1 shows the modeling domains. Two-nested domains are used in WRF and CMAQ simulations with 31 vertical levels. The horizontal resolutions are 27km and 9km in coarse domains and inner domains, respectively. The exterior grids of CMAQ model cover most regions of China, and the interior grids cover the whole Guangdong Province. The locations of major cities in PRD are given in Figure 1 too. The simulations are conducted for the period from 08:00 LST (local standard time) 25 Dec. 2013 to 08:00 LST 01 Feb. 2014. The first seven days are used for model spin-up runs.

Emissions are very important to the air quality simulations. Many other studies have devoted to improve the emission inventory in China [31-32]. This study uses the 2010 Multi-resolution Emission Inventory for China (MEIC) with horizontal resolutions of 0.25°×0.25°. The MEIC was originally developed by Tsinghua University China [33] and represents the latest update of emission inventory in China. Emissions of MEIC are divided into five categories. They include the emissions from power plants, industry, agriculture, transportation, and resident. The monthly mean emissions of gas pollutants such as SO2, NOX, CO and NH3, aerosol chemical compositions such as sulfate, nitrate, ammonium, black carbon, organic carbon, and other substances are used in this study.

Figure 2 shows the spatial distributions of annual emissions of NO2, NO, SO2 and CO of Guangdong province. It is found that major cities are the most important emissions. For example, more than 500×106mol of CO、NO, and 50×106mol of SO2、NO2 are emitted by Guangzhou, Foshan and Shenzhen per year. Also the temporal variations of the MEIC dataset are given by Zheng [34].

a)

b)

c)

d)

Figure 2. Annual emissions of a) NO2, b) NO, c) SO2and d) CO from MEIC dataset (Unit: ×106mol/year).

2.2 Contributions of emissions from different regions

To quantify the relative contributions of local and different regional emissions, the emissions are divided into four different parts: local emissions from Zhongshan, emissions from other cities in PRD, emissions outside Guangdong, and emissions outside PRD in Guangdong. Four scenarios are designed to evaluate the contributions of different emissions to the PM2.5 concentrations by turning off the emissions individually. The details of different emissions are shown in Figure 1 and Table 1 (emissions outside Guangdong Province are not showed here). The first scenario, Test_ctr is the control simulation which represents the benchmark with original emissions. The second scenario, Test_zs represents the case when the local emissions of Zhongshan are turned off. The third scenario, Test_prd is the case when the emissions of other cities in PRD are turned off; this case is used to evaluate the influence of emissions from surrounding regions on Zhongshan. The last scenario, Test_gd is the case when the emissions outside Guangdong Province are turned off. The contribution of each region can be calculated by using the following formula:

Cx=Cctr-Cx,0           (1)

Px=Cx/Cctr                  (2)

, where Cctr represents the PM2.5concentration under the benchmark condition, Cx,0is the PM2.5concentrationwhen the emissions from region x are set to zero,Cx represents the difference of PM2.5concentrationsbetween emissions are turned on and off in region x, Px represents the contribution of emissions inregion x.The similar method has been used by other studies during 2008 Beijing Olympic Game [35-36].

Table1. A summary of experiment simulations.

| Experiment | Emission to be turned off |
| --- | --- |
| Test_ctr | None |
| Test_zs | Zhongshan |
| Test_prd | PRD (excluding Zhongshan) |
| Test_gd | Outside Guangdong Province |

3. Evaluation of meteorological and air quality model simulations

3.1 Evaluation of meteorological simulations

Meteorological inputs are important for air quality modeling study. Surface observational data at the sites of Zhongshan (113.35°E，22.53°N), Guangzhou (113.33°E，23.17°N) and Zhuhai (113.57°E，22.28°N) are used to verify the WRF simulations. Figure 3 shows the comparisons of WRF simulated daily mean 2m temperature, daily mean relative humidity, 24-hour surface pressure difference and daily mean wind speed with observational data in January 2014. Here the surface pressure difference denotes the difference of surface pressure between current day and one day before. The positive difference in winter indicates an intrusion of a cold front. It is seen from Figure 3 that the simulated 2m temperature at the three sites is slightly higher than the actual temperature in most of the time but the temporal variation patterns match the actual data quite well. The relative humidity is a little lower than the actual values. The simulated 24-hour surface pressure differences show excellent agreement with observations in the three sites. It can be seen from the variations of temperature and surface pressure that there were six evident cooling periods associated with cold front activities, namely 3-5, 8-9, 12-13, 18-19, 21-22, and 26-27 January. The wind speeds are over-predicted for most days, especially for the days with substantial impact from cold fronts (e.g. 8, 18, and 21 January), but the tendency of daily variations fits the actual values quite well. Overall, the simulations show that the WRF model is able to provide reasonable meteorological inputs for air quality modeling in January 2014.

a)

Zhongshan             Guangzhou              Zhuhai

b)

c)

d)

Figure 3 Comparison of simulated a) daily mean 2m temperature, b) daily mean relative humidity, c) 24-hour surface pressure difference and d) daily mean wind speed with the observations (black lines for observation, and red lines for simulation).

3.2 Evaluation of CMAQ model results

Figure 4 shows the time series comparison of PM2.5 concentration and visibility between the model simulations and observations in Zhongshan in January 2014. The hourly air quality observational data are provided by Zhongshan Protection Bureau. Currently there are four observation sites in Zhongshan, which are not far away from each other. So data from those four sites are averaged for model evaluation as well as the model output of grids where the four sites located. As seen in Figure 4, overall the model is able to capture the general temporal variation patterns of PM2.5 concentrations. However, the model under-predicts the high PM2.5 events substantially and sometime over-predicts the PM2.5. The simulation biases could be related to the uncertainties of emissions and the WRF model simulations. For example, when no cold front appears, the simulated wind speeds are generally stronger than the observations. As a result, the simulated PM2.5 could be lower than the observations. 5 and 23 January are the two examples for that. When a strong cold front moves southward, it transports a large amount of air pollutants to the downwind areas. If the cold front is strong enough, the pollutants can be transported over a longer distance. If the simulated cold front is weaker than the observation (i.e., the simulated 24-hour surface pressure difference is smaller than observation), the model will weaken the transport of extraneous pollutants, and further reduce the local PM2.5concentration. A typical example can be seen on 12 January. Another example is found during the Chinese New Year eve from 28 to 30 January. The air pollutant emissions were less than the normal level in those days. However, the model fails to capture such change, causing an obvious difference between the simulation and the observation. The simulated PM2.5 concentrations are much higher than the observations during those three days. Especially the model was not able to capture the PM2.5 peak value on the Chinese New Year (i.e, 31st January). This is mainly because that the firework emissions were not included in the current emission inventory which is used in the simulations.

Meanwhile, the model is able to capture the variation pattern of visibility. However, the visibility is over-predicted when no clod front appears. This is because the PM2.5 is under-predicted in this case. Further, the simulated visibility is lower than the observation on 28-30 January because the simulated PM2.5 concentration is overestimated. The difference is the most obvious when the observed or the simulated visibility reaches their peak values. Overall the modeling system shows similar performance for both visibility and PM2.5 since the former is highly dependent on the latter.

a)

b)

Figure 4.Comparison of simulated a) PM2.5 concentration and b) visibility with observations (the discontinuities are due to the missing observational data).

Table2.The evaluation statistics of CMAQ performance on PM2.5 (g·m-3)and visibility(km).

|  | MB | ME | RMSE | MNB | NMB |
| --- | --- | --- | --- | --- | --- |
| PM2.5 Concertration | -13 | 32 | 29 | -4.0% | -21.4% |
| Visibility | 0.8 | 4.9 | 5.6 | 8.2% | 20.2% |

Table 2 shows the evaluation statistics of the CMAQ performance on PM2.5 and visibility simulations. Several statistical metrics are used to evaluate the model performance. They include mean bias (MB), mean absolute error (MAE), root mean square error (RMSE), mean normalized bias (MNB), and normalized mean bias (NMB). They are defined as follows:

MB=1ni=1nSimi-Obsi                   (1)

MAE=1ni=1n|Simi-Obs(i)|                  (2)

RMSE=[1ni=1n(Simi-Obs(i))2]1/2            (3)

MNB=1ni=1n(Simi-Obs(i)Obs(i))                      (4)

NMB=i=1n(Simi-Obs(i))i=1nObs(i)                        (5)

, where Sim and Obs represent simulated and observed values, respectively, and n is the number of observational sites. The calculated statistical parameters for PM2.5 and visibility are presented in Table 2. It is found from Table 2 that the simulated PM2.5 concentrations are lower than the observations and the simulated visibility is slightly over-predicted. Overall the statistical calculations indicate that the simulatedPM2.5 and visibility show reasonable agreement with the observations.

4. Sensitivity analysis

4.1 Contributions of different regional emissions

b)

a)

d)

c)

Figure 5. a) Spatial distribution of simulated monthly mean PM2.5 concentration; b) percentage contributions of local emissions in Zhongshan; c)percentage contributions of emissions from other cities in the PRD, and d) percentage contributions of emissions outside Guangdong Province.

Figure 5a shows the spatial distribution of monthly mean simulated PM2.5 concentrations. A relatively high concentration of PM2.5 plume is found in the middle of Foshan (locations of the cities can be found in Figure1b), where the monthly mean PM2.5 concentrations exceed 60μg/m3. PM2.5 concentration in the northwest of Zhongshan is 50-55μg/m3, and only 35-40μg/m3is found in the southern part. The PM2.5 from northwestern to southeastern of Zhongshan is gradually reduced. Figure 5b illustrates the contributions of local emissions in Zhongshan. The contribution reaches 45% in the northwest of Zhongshan but reduces to 30-35% in the southern area. The downwind of Zhongshan, including part of Jiangmen and Zhuhai, are also affected by the emissions from Zhongshan. Figure 5c shows the contributions of emissions in other cities of PRD (see Figure 1b). Evidently the contribution in the middle and southeast of Foshan exceeds 60%. Contributions in southern Guangzhou, western Dongguan and Shenzhen, and eastern Jiangmen are higher than 50%. These emissions contribute 35-45% of PM2.5 in northern and 25-30% in southern Zhongshan. In addition, the emissions outside of Guangdong Province have important impact on of the PRD (see Figure 5d). These parts of emissions contribute relatively less nearby the southeast of Foshan, but the contribution gradually increases in other parts of PRD. To Zhongshan, emissions outside Guangdong contribute about 25% in the northern part but the contribution increase to 35-40% in southern areas. Therefore, the emissions of local Zhongshan, other cities in the PRD, and outside Guangdong Province are all important to PM2.5 in Zhongshan.

c)

b)

a)

Figure 6. Percentage contributions of the emissions from different regions to the simulated PM2.5 in Zhongshan under a) no cold front, b) strong cold front and c) weak cold front case.

4.2 Regional transport with different weather conditions

The weather conditions play an important role in regional air pollutant transport. Figure 6 shows the percentage contributions by different regional emissions under different weather conditions in Zhongshan. When there is no cold front intrusion (i.e., 24-hour surface pressure difference is negative), haze pollution is mainly caused by low wind speeds and stable atmospheric boundary layer condition. Meanwhile, such weather condition is not beneficial to long-distance transport of air pollutants. So the contributions of local emissions, from nearby cities in PRD, from outside Guangdong and from other emissions in Guangdong to the PM2.5 in Zhongshan are 58%, 27%, 13% and 2% respectively. When a strong cold front (24-hour surface pressure difference is more than 3hPa) moves southwards, a large amount of non-local pollutants from outside of Guangdong Province are transported to the PRD region. The contribution of the emissions from outside Guangdong Province is increased to 45% in this case, followed by local emissions of Zhongshan (28%), and other cities in the PRD (23%). When a weak cold front (24-hour surface pressure difference is about 0-3hPa) appears, the air pollutants are mainly controlled by short-distance transport. The contribution of emissions from other cities in the PRD increase to 42%, followed by local emissions of Zhongshan (34%), and the emissions from outside of Guangdong Province (21%)

It can be found from the results above that emissions contributed differently to the haze pollution of Zhongshan under different weather conditions. Additionally, the simulated wind speed is usually higher than actual conditions. So the cross-region transportation of air pollutants may be overestimated to some extent, and the pollution in low wind speed cases may be weakened. Therefore, the model results may overestimate the contribution of non-local emissions, but underestimate the local one.

Table3. Changes of visibility due to turning off different emissions under different weather conditions in Zhongshan.

| The regions where emissions are turned off | No cold front (%) | Weak cold front (%) | Strong cold front (%) |
| --- | --- | --- | --- |
| Zhongshan | 94 | 39 | 30 |
| Other cities in PRD | 30 | 76 | 27 |
| Outside Guangdong | 15 | 23 | 84 |

Now we turn our attention to the visibility. Table 3 presents the changes of visibility under different weather conditions when the emissions in different regions are turned off. It is seen from the table that the visibilities are increased for all the scenarios. The visibility shows the largest increase (94%) for the case when the emissions in Zhongshan are turned off and there is no cold front intrusion. The next large increase can be happened when the emissions from outside for Guangdong are turned off and a strong cold front appears in this region. For other cases, the changes of visibility vary from 15% to 76%. Thus, the changes of emissions in different regions have substantial impact on the visibility. These impacts are highly dependent on the weather conditions.

d)

c)

a)

b)

Figure 7. The spatial distribution of simulated PM2.5 (μg/m3) and 10-m wind fields (m/s) at a) 08:00 LST (Local Standard Time); b) 17:00 LST; c) 23:00 LST on 8 January, and d) 17:00 LST on 9 January, 2014.

According to the analysis above, it can be found that emissions outside Guangdong Province contribute most changes of local PM2.5 concentration in Zhongshan when strong cold front affects the PRD region. Figure 7 shows the impact of regional transport on PM2.5 in Zhongshan during 8-9 January. It is found that the most polluted area was located in Hunan Province on 8 January. As the cold front moves southwards, the aerosol pollutants were transported southwards and caused large increase of PM2.5 concentration in northern Guangdong. On the night of 8 January, a large amount of pollutants were transported to the PRD region and the western part of Guangdong. By the afternoon of 9 January, the PM2.5 concentration in the PRD started to decrease. Meanwhile, PM2.5 concentrations were decreased due to the diffusion, sedimentation, and aging. Similar transport processes of pollutants were also observed for other cold front cases. Thus regional transport of air pollutants from outside Guangdong Province poses a large impact on the local haze pollution in Zhongshan during winter.

4.3 Impact of local emission controls at different stages

According to the analysis above, the emission control has the most effect when there is no cold front intrusion or the regional transport is not important. The effectives of local emission become much less when there is a strong cold front intrusion. In this section, we are focused on the impact of local emission control on the different stages.

Table 4 shows the changes of PM2.5 in Zhongshan on different days after local emissions are turned off for the scenario when there is no cold front intrusion. It is clear that emission control has different impact on the different days. In specific, PM2.5 concentrations were decreased by 47%, 52% and 58% on the first, second and third day respectively after the local emissions were turned off in Zhongshan. It is apparent that emission control has clear phase effect. Similar phase effect is also observed for the changes of the local visibility which was increased by 57%, 63%, and 90% on the first, second, and third days, respectively. Therefore the large impact of local emission on PM2.5 and visibility tends to appear on the second and third day after the local emission are turned off. This suggests that the emission control can be implemented on 2-3 days before a haze episode appears based on air quality forecasting.

Table 4. Percentage change of simulated PM2.5 concentration and visibility when local emissions are turned off in Zhongshan.

|  | PM2.5 concentration (%) | Visibility (%) |
| --- | --- | --- |
| 1st day | -47 | 57 |
| 2nd day | -52 | 63 |
| 3rd day | -58 | 90 |

4.4Impact of different local anthropogenicemission controls

Industrial, residential, and mobile emissions are the three major emissions to local PM2.5 in Zhongshan. Now we evaluate the impact of the three emissions on PM2,5 and visibility in Zhongshan during a heavy haze pollution event. Here we evaluate their impact on PM2.5 during a peak period on the third day after the emission control starts. It can be found from Table 5 that the industrial emission has the largest impact (-24%), the mobile emission has the smallest impact (-15%), and the residential emission stands between them when the three emissions are turned off individual. Meanwhile, the visibility is increased by 26%, 21% and 16%, respectively. Therefore, the government should consider how to control different emissions for more efficient reduction of the occurrence and severity of heavy haze events.

Table 5. Percentage change of simulated PM2.5 concentration and visibility during heavy polluted periods after turning off industrial, residential and mobile emissions in Zhongshan.

| Emissions to be turned off | PM2.5 (%) | Visibility (%) |
| --- | --- | --- |
| Industrial sources | -24 | 26 |
| Residential sources | -22 | 21 |
| Mobile sources | -15 | 16 |

5. Conclusions

Scientific information on the contribution of emission sources is very important to the control of haze pollution but such studies are still not enough in Zhongshan so far. So the haze pollution over Zhongshan during January 2014 is simulated using the WRF-CMAQ model in this paper. The main conclusions, which can be used as a reference for haze control for government, are as follows:

The simulation results based on WRF-CMAQ model can reflect the evolution of the meteorological factors and the PM2.5 concentration of Zhongshan. PM2.5 concentration of Zhongshan gradually reduces from northwestern to southeastern. During January 2014, the contribution of local emission sources reaches 45% in the northwest of Zhongshan but reduces to 30-35% in the southern area. Emission sources of other cities in RPD contribute 35-45% PM2.5 in northern Zhongshan but reduce to 25-30% in the southern area. The contribution of emission sources outside Guangdong Province is 25% in the northern part but reaches 35-40% in southern Zhongshan.

The simulation tests show that the local emissions contribute most (58%) to the PM2.5 of Zhongshan under no cold front case. However when a strong cold front moves southward, a large amount of pollutants outside Guangdong Province are transported to the PRD region. That makes the contribution of emissions outside Guangdong increases to 45%. When under weak cold front case, emissions of other cities in PRD contribute 42%, followed by the local emissions and emissions outside Guangdong.

Results of the emission control tests show that after turning off the whole local emission sources, PM2.5 concentration of Zhongshan is decreased by 47%, 52% and 58% on the first, second and third day. So emission controls are suggested to be implemented 2-3 days before haze days to reduce PM2.5 by 50%. The haze pollution of Zhongshan mostly come from industrial sources, followed by residential and transportation sources. After turning off industrial, residential and transportation sources, PM2.5 concentration is decreased by 24%, 22% and 15%, respectively.

Acknowledgments

This study is supported by the Key Projects in the National Science & Technology Pilar Program (2014BAC16B06), Project supported by the National Natural Science Foundation of China (41205123), Science & Technology Research Project of Guangdong Meteorological Service, China (2014B32), Special Research Project of Public Service Sectors (GYHY201306042). Also, the emission source inventory used in this paper is supplied by Center For Earth System Science from Tsinghua University, China. The authors would like to thank all people for their great work which helped to improve this manuscript.

References

[1]T. Wang, X. L. Wei, A. J. Ding, C. N. Poon, K. S. Lam, Y. S. Li, L. Y. Chan, and M. Anson. “Increasing surface ozone concentrations in the background atmosphere of southern China, 1994-2007”, Atmospheric Chemistry and Physics Discussions, vol. 9, no. 16, pp. 10429-10455, 2009.

[2] Y. C. Li, J. Z. Yu, S. H. H. Steven, Z. B. Yuan, A. K. H. Lau, and X. F. Huang, “Chemical characteristics of PM2.5 and organic aerosol source analysis during cold front episodes in Hong Kong, China”, Atmospheric Research, vol. 118, pp.41-51, 2012.

[3]X. X. Tie, D. Wu, and B. Guy, “Lung cancer mortality and exposure to atmospheric aerosol particles in Guangzhou, China”, Atmospheric Environment, vol. 43, no. 14, pp. 2375-2377, 2009.

[4]D. Wu, X. X. Tie, C. C. Li, Z. M. Ying, A. K. Lau, J. Huang, X. J. Deng, and X. Y. Bi, “An extremely low visibility event over the Guangzhou region: A case study”, Atmospheric Environment, vol. 39, no. 35, pp. 6568-6577, 2005.

[5]Q. Lu, J. Y. Zheng, S. Q. Ye, X. L. Shen, Z. B. Yuan, and S. S. Yin, “Emission trends and source characteristics of SO2, NOX, PM10 and VOCs in the Pearl River Delta region from 2000 to 2009”, Atmospheric Environment, vol.76, pp. 11-20, 2013.

[6]X. J. Deng, X. X. Tie, D. Wu, X. J. Zhou, X. Y. Bi, H. B. Tan, F. Li, and C. L. Jiang, “Long-term trend of visibility and its characterizations in the Pearl River Delta Region(PRD), China”,  Atmospheric Environment, vol. 42, no. 7, pp. 1424-1435, 2008.

[7]H. Huang, K. F. Ho, S. C. Lee, P. K. Tsang, S. S. H. Ho, C. W. Zou, S. C. Zou, J. J. Cao, and H. M. Xu, “Characteristics of carbonaceous aerosol in PM2.5: Pearl Delta River Region, China”, Atmospheric Research, vol. 104-105, pp. 227-236, 2012.

[8] P. F. Liu, C. S. Zhao, T. Gobel, E Hallbauer, N. Andreas, and L. Ran, “Hygroscopic properties of aerosol particles at high relative humidity and their diurnal variations in the North China Plain”, Atmospheric Chemistry and Physics, vol. 11, pp. 3479-3494, 2011.

[9]D. Wu, X. X. Tie, and X. J. Deng, “Chemical characterizations of soluble aerosols in Southern China”, Chemosphere, vol. 64, no. 5, pp. 749-757, 2006.

[10]J. H. Tan, S. J. Guo, Y. L. Ma, J. Duan, Y. Cheng, K. He, and F. Yang, “Characteristics of particulate PAHs during a typical haze episode in Guangzhou, China”, Atmospheric Research, vol. 102, pp. 91-98, 2011.

[11]D. Wu, J. T. Mao, X. J. Deng, X. X. Tie, Y. H. Zhang, L. M. Zeng, F. Li, H. B. Tan, and X. Y. Bi, “Black carbon aerosols and their radiative properties in the Pearl River Delta region”, Science in China Series D: Earth Sciences, vol. 52, no. 8, pp. 1152-1163, 2009.

[12]H. J. Xu, X. M. Wang, U. Poschl, S. L. Feng, D. Wu, Y. Ling, S. X. Li, W. Song, G. Y. Sheng, and J. M. Fu, “Genotoxicity of total and fractionated extractable organic matters in fine air particulate matter (PM2.5) in unban Guangzhou：comparison between haze and non-haze episode days”, Environmental Toxicology and Chemistry, vol. 27, no. 1, pp. 206-212, 2008.

[13]H. Yu, C. Wu, D. Wu, and J. Z. Yu, “Size distributions of elemental carbon and its contribution to light extinction in urban and rural locations in the Pearl River Delta, China”, Atmospheric Chemistry and Physics, vol. 10, no. 12, pp. 5107-5119, 2010.

[14]C. K. Chan, and X. H. Yao, “Air pollution in mega cities in China”, Atmospheric Environment, vol. 42, pp. 1-42, 2008.

[15]G. S. W. Hagler, M. H. Bergin, L. G. Salmon, J. Z. Yu, E. C. H. Wang, M. Zheng, L. M. Zeng, C. S. Kiang, Y. H. Zhang, A. K. H. Lau and J. J. Schauer, “Source areas and chemical composition of fine particulate matter in the Pearl River Delta region of China”, Atmospheric Environment, vol. 40, 3802-3815.

[16]A. P. Kesarkar, M. Dalvi, A. Kaginalkar, and A. Ojha, “Coupling of the weather research and forecasting model with AERMOD for pollutant dispersion modeling: A case study for PM10 dispersion over Pune, India”, Atmospheric Environment, vol. 41,no. 9, pp. 1976-1988, 2007.

[17]P. Pongkiatkul, and N. T. K. Oanh, “Assessment of potential long-range transport of particulate air pollution using trajectory modeling and monitoring data”, Atmospheric Research, vol. 85, pp.3-17, 2007.

[18]W. P. Shan, Y. Q. Yin, H. X. Lu, and S. X. Liang, “A meteorological analysis of ozone episodes using HYSPLIT model and surface data”, Atmospheric Research, vol. 93, pp.767-776, 2009.

[19]J. M. Wan, L. M. Lin, C. Y. Chan, Z. S. Zhang, G Engling, X. M. Wang, I. N. Chan and S. Y. Li, “Change of air quality and its impact on atmospheric visibility in central-western Pearl River Delta”, Environmental monitoring and assessment, vol. 172, pp. 339-351, 2011.

[20] Q. Fan, J. Lan, Y. M. Liu, X. M. Wang, P. Chan, S. J. Fan, Y. Y. Hong, Y. X. Liu, Y. J. Zeng, G. X. Liang, and Y. R. Feng. “Diagnostic analysis of the sulfate aerosol pollution in spring over Pearl River Delta, China”, Aerosol and Air Quality Research, vol. 15, pp. 46-57, 2015.

[21]W. W. Che, J. Y. Zheng, S. S. Wang, L. Zhong, and A. Lau, “Assessment of motor vehicle emission control policies using Model-3/CMAQ model for the Pearl River Delta region, China”, Atmospheric Environment, vol. 45, no. 9, pp. 1740-1751, 2011.

[22]R. Zhang, S. Golam, C. H. F Jimmy and K. H. L. Alexis, “Role of photoexcited nitrogen dioxide chemistry on ozone formation and emission control strategy over the Pearl River Delta, China”,  Atmospheric Research, vol. 132-133, pp. 332-344, 2013.

[23]Y. R. Feng, A. Y. Wang, D. Wu, and X. D. Xu, “The influence of tropical cyclone Melor on PM10 concentrations during an aerosol episode over the Pearl River Delta region of China: Numerical modeling versus observational analysis”, Atmospheric Environment, vol. 41, no. 21, pp. 4349-4365, 2007.

[24]F. Jiang, T. J. Wang, T. T. Wang, and M. Xie, “Numerical modeling of a continuous photochemical pollution episode in Hong Kong using WRF-chem”, Atmospheric Environment, vol. 42, no. 38, pp. 8717-8727, 2008.

[25]X. S. Wang, Y. H. Zhang, Y. T. Hu, W. zhou, L. M. Zeng, M. Hu, D. S. Cohan, and A. G. Russell, “Decoupled direct sensitivity analysis of regional ozone pollution over the Pearl River Delta during the PRIDE-PRD 2004campaign”, Atmospheric Environment, vol. 45, no. 28, pp. 4941-4949, 2011.

[26]L. J. Zhong, P. K. K. Louie, J. Y. Zheng, Z. B. Yuan, D. L. Yue, J. W. K. Ho, and A. K. H. Lau, “Science-policy interplay: Air quality management in the Pearl River Delta region and Hong Kong”, Atmospheric environment, vol. 76, pp. 3-10, 2013.

[27]H. Liu, X. M. Wang, J. M. Pang, and K. B. He, “Feasibility and difficulties of China’s new air quality standard compliance: PRD case of PM2.5 and ozone from 2010 to 2025”, Atmospheric Chemistry and Physics, vol. 13, pp. 12013-12027, 2013.

[28]W. C. Skamarock, J. B. Klemp, J. Dudhia, D. O. Gill, D. M. Barker, and M. G. Duda. “A description of the advanced research WRF version 3”. NCAR Tech. Note, NCAR/TN-475+STR, pp. 1-5,2008.

[29] N. Wang, H. Guo, F. Jiang, Z. H. Ling, and T. Wang, “Simulation of ozone formation at different elevations in mountainous area of Hong Kong using WRF-CMAQ model”, Science of the Total Environment, vol. 505, pp. 939-951, 2015.

[30]W. T. Hutzell, D. J. Luechen, K. W. Appel, W. P. L. Carter, “Interpreting predictions from the SAPRC07 mechanism based on regional and continental simulations”, Atmospheric Environment, vol. 46, pp. 417-429, 2012.

[31]Q. Zhang, D. G. Streets, G. R, Carmichael,  K. B. He, H. Huo, A. Kannari, Z. Klimont, I. S. Park, S. Reddy, J. S. Fu, D. Chen, L. Duan, Y. Lei, L. T. Wang, and Z. L. Yao, “Asian emissions in 2006 for NASA INTEX-B mission”, Atmospheric Chemistry and Physics, vol. 9, pp. 5131-5153, 2009.

[32]Q. Zhang, G. N. Geng, S. W. Wang, A. Richter, and K. B. He, “Satellite remote sensing of changes in NO(x) emissions over China during 1996-2010”, Chinese Science Bulletin, vol.  57, pp. 2857-2864, 2012.

[33]K. B. He, “Multi-resolution Emission Inventory for China (MEIC): model framework and 1990-30 2010 anthropogenic emissions”, International Global Atmospheric Chemistry Conference, 2012.

[34]J. Y. Zheng, Z. Y. Zheng, Z. L. Wang, L. J. Zhong, and D. Wu, “Biogenic VOCs emission inventory and its temporal and spatial characteristics in the Pearl River Delta area”, China Environmental Science, vol.29, no. 4, pp. 345-350, 2009.

[35]Wang, L. T. Wang, J. M. Hao, He, K. B. He, S. X. Wang, J. H. Li, Q. Zhang, D. G. Streets, J. S. Fu, C. J. Jang, H. Takekawa, and S. Chatani , “A modeling study of coarse particulate matter pollution in Beijing: region source contributions and control implications for the 2008 Summer Olympics”, Journal of Air and Waste Management Association, vol. 58, pp. 1057-1069, 2008.

[36]J. Xing, Y. Zhang, S. X. Wang, X. H. Liu, S. H. Cheng, Q. Zhang, Y. S. Chen, D. G. Streets, C. Jang, J. M. Hao, and W. X. Wang, “Modeling study on the air quality impacts from emission reductions and a typical meteorological conditions during the 2008 Beijing Olympics”, Atmospheric Environment, vol. 45, pp. 1786-1798, 2011.