A modeling study of impact of emission sources on haze in  Zhongshan China using WRF-CMAQ

Jianhua Mai1,2, Tao Deng2, Lingling Yu3, Xuejiao Deng2, Haobo Tan2, Shiqiang Wang4, Xiantong Liu2

1 Zhongshan Meteorological Service, Zhongshan 528400, China

2 Institute of Tropical and Marine Meteorology/Guangdong Provincial Key Laboratory of Regional Numerical Weather Prediction, Guangzhou, China Meteorological Administration, Guangzhou 510080, China

3 Guangdong Meteorological Observatory, Guangzhou 510080, China

4 Zhuhai Meteorological Service, Zhuhai 519000, China

Corresponding Author: Tao Deng

Institution: Institute of Tropical and Marine Meteorology/Guangdong Provincial Key Laboratory of Regional Numerical Weather Prediction, Guangzhou, China

Address: 6 Fujin Road, Yuexiu, Guangzhou, China

Tel: +86 13560376450

E-mail: tdeng@grmc.gov.cn

Abstract: The WRF-CMAQ modeling system is used to study the impact of emission control strategies and weather conditions on  haze pollution in Zhongshan, Guangdong Province, China during January 2014. Overall  the modeling system is able toreproduce the meteorological parameters and PM2.5 concentration quite well. Several sensitivity studies are completed to quantify the relative percentage contributions of local emissions versus regional emissions to the PM2.5 concentrations under different weather conditions. The results indicate that the contribution of local emissions  from Zhangshan is higher than that of the emissions from outside of the region when no cold fronts appear in the Pearl River Delta (PRD) region (i.e., 58% contribution from local emission versus 42% contribution from the regional transport).  .However, the contribution of the emissions from the regional transport is increased to 45% when a strong cold front appears.   Furthermore, the sensitivity study shows that According to the emission control tests, PM2.5 concentration of Zhongshan can be reduced by 47%, 52% and 58% on the first, second and third day after turning off the whole local emission sources under no cold front weather conditions. And the industrial, residential and transportation sources of Zhongshan contribute 24%, 22% and 15% PM2.5 over the local area during the heavy polluted period, respectively.

Keywords: WRF-CMAQ, Haze, Contribution ratio, Emission control, PM2.5

1. Introduction

Along with the rapid growth of the economy and massive emissions of atmospheric pollutants, haze is tending to become the most important pollutant event to air pollution in China [1]. It is potentially harmful to manufacture, transportation and even human health [2-3]. As one of the three biggest city clusters in China, the Pearl River Delta (PRD) is the fastest growing economic area and therefore inevitably suffered from deteriorating air quality, such as haze pollutions. The observed data shows that the averaged number of haze days was over 100 in recent years and the air pollution control in PRD is of great urgency [4]. In order to improve air quality in PRD, the government has made great efforts to formulate policies about emission controls and to some extent, these efforts have already helped in the contribution of improving air quality in PRD. One aspect can be seen on the haze issue [5-9]. Lots of scholars focused on studying the composition, weather characteristics and meteorological effects of haze [10-13]. With the rapid development of computer, numerical models have become important tools for atmospheric environmental research [14]. For example, the Hysplit trajectory model is mostly used to study the trajectory of airflow affecting certain areas [15-16]. The CMAQ (Community Multi-scale Air Quality) model, well known as a numerical tool for environment researching, is often used to study pollution events happened over PRD [17-18]. Currently, quite a number of studies based on numerical models are focused on the evolution and influence of the atmospheric pollution process [19-21]. But more importantly, some research show that haze pollution events in PRD are typically caused by emission sources of varying spatial scales under specific weather conditions [22]. However, little attention had been paid on the sources of atmospheric aerosols in the PRD region and the contribution ratio of emission sources so far. Without systematic evaluations of emission sources based on scientific methods, it is hard to make better strategies to improve air quality [23]. Therefore, further research is still needed in order to formulate more effective control strategies in PRD.

Zhongshan City, an important part of PRD, has been suffering from frequent haze pollution in recent years. However, the lack of large-scale PM2.5 monitoring and efficient control strategy is the main obstacles for getting rid of haze pollution [24]. Thus, in this study we aim to providing some scientific evidences for local policy-makers to set up more effective emission controls. By taking advantage of the observed data and the WRF-CMAQ model, a numerical analysis is performed about haze pollution over Zhongshan during January 2014, a month in which there were 16 haze days. The contributions of emission sources from different areas to the local PM2.5 concentration of Zhongshan as well as the evaluation of emission controls can be learnt from this paper.

2. Methods

2.1 Descriptions of models and simulation configurations

This paper uses the WRF-CMAQ model [25-26] to perform numerical simulations. The WRF model (Weather Research and Forecasting, version 3.3.1) [27] provides detailed meteorological data for CMAQ and 6-hourly Final Operational Global Analysis data with 1°×1° resolution from the National Center for Environmental Prediction (NCEP) are used for initial and boundary conditions. The CMAQ model (version 5.0.2) is a multi-scale Euler air quality model which combines current knowledge in atmospheric science and air quality modeling. It can simulate the physical and chemical process of multiple pollutants in the atmosphere. The CMAQ model must be driven by the results of meteorological models and the emission source data. It comprehensively considers the chemistry, transport processes and air-surface exchange of the air pollutants. More details for CMAQ can be found in Binkowski‘s paper [28]. The carbon bond (CB05) gas-phase chemical mechanism and the AERO5 module for aerosol chemistry are used in our simulation. In this study, the default boundary condition of CMAQ is used in Domain_1 of CMAQ. The Numerical tests use a two-way nested method. As shown in Table 1 and Figure 1, the exterior grids of CMAQ model cover most regions of China, and the interior grids cover the whole of Guangdong Province. The locations of cities in PRD are showed in Figure 1 too. Simulation period is from 08:00 LST (local standard time) 25 Dec. 2013 to 08:00 LST 01 Feb. 2014. The days before 08:00 LST 01 Jan. 2014 are for model spin-up.

Table 1. Setting of the simulation domains.

| Simulation domains | Number of grid points | Grid space (km) |
| --- | --- | --- |
| WRF Model Domain_1 | 283×184 | 27 |
| CMAQ Model Domain_1 | 182×138 | 27 |
| WRF Model Domain_2 | 233×163 | 9 |
| CMAQ Model Domain_2 | 98×74 | 9 |

b)

a)

Figure 1. a) Two-nested modeling domains for WRF (black solid boxes) and CMAQ ( blue dashed boxes) models; b) different parts of emission sources in different colors (details will be given in Section2.2) (Cities of PRD in the right picture: Zhongshan (ZS), Guangzhou (GZ), Foshan (FS), Jiangmen (JM), Zhuhai (ZH), Dongguan (DG) and Shenzhen (SZ)).

Emission sources are very important to the air quality model. Many scholars in China try their best to improve the emission inventory [29-30]. The emission data used by this paper come from the 2010 Multi-resolution Emission Inventory for China (MEIC) with 0.25°×0.25° resolution developed by Tsinghua University. The MEIC is the best emission inventory in China currently, which is deal with a spatial interpolation method to fit the simulation. Emission data in MEIC is divided into five categories, i.e.: power plants, industry, agriculture, transportation and resident. It covers the monthly emissions [31] of gas pollutant such as SO2, NOX, CO and NH3, aerosol such as PM2.5 and PM10, as well as black carbon, organic carbon and other substances. Figure 2 shows the annual emissions of NO2, NO, SO2 and CO of Guangdong province from the MEIC dataset. It is found that the PRD are regions with the most concentrated pollutant emissions. More than 500×106mol of CO、NO, and 50×106mol of SO2、NO2 are discharged from Guangzhou, Foshan and Shenzhen per year. Also the temporal variations of the MEIC dataset are described by Zheng [32].

a)

d)

c)

b)

Figure 2. Annual emission of a) NO2, b) NO, c) SO2 and d) CO of Guangdong Province from MEIC dataset (Unit:×106mol/year).

2.2 Contributions of emissions from different regions

In order to evaluate the influence of emission sources inside and outside Zhongshan on the local haze pollution, the WRF-CMAQ model is used to study the contributions of emissions from different regions. To figure out the contribution ratios, emission sources in the model are divided into four parts (sources of local Zhongshan, sources of other cities in PRD, sources outside Guangdong Province, and sources outside PRD in Guangdong). And then we turn off the emission sources from a certain region and compare the simulation results among tests. At last we calculate the contribution ratio of the emission sources from this region. The research method was proved to be effective in some studies on the air quality during 2008 Beijing Olympic Game [33-34]. The specific test schemes and different parts of emission sources are shown in Figure 1 and Table 2 (emission sources outside Guangdong Province are not showed here). Test_ctr is the control test which represents the benchmark conditions of emission sources; in Test_zs the local emission sources of Zhongshan are turned off; in Test_prd the emission sources of other cities in PRD are turned off so as to evaluate the influence of emission sources from surrounding regions on Zhongshan; in Test_gd the emission sources outside Guangdong Province are turned off. By excluding the influences from local Zhongshan, the PRD and those from outside Guangdong, the remaining air pollutants are from other parts of Guangdong (see Figure 1). The contribution ratio of a certain region can be calculated by the following formula:

Cx=Cctr-Cx,0               (1)

Px=Cx/Cctr                 (2)

where Cctr and Cx,0 represent the PM2.5 concentration under the benchmark condition and the condition that the emissions from region x are set to zero, respectively; Cx represents the difference of PM2.5 concentrations before and after emission sources are turned off in region x; Px represents the contribution ratio of emission sources in region x.

Table 2. A summary of simulation tests.

| Test name | Emission source to be turned off | Test name | Emission source to be turned off |
| --- | --- | --- | --- |
| Test_ctr | None | Test_zs | Zhongshan |
| Test_prd | PRD (excluding Zhongshan) | Test_gd | Outside Guangdong |

3. Evaluation of meteorological and air quality model simulations

3.1 Evaluation of meteorological simulations

The simulation accuracy of the CMAQ model depends on the results of the meteorological simulations. Therefore the results of WRF model should be first evaluated. Observed data used in this section are from sites of Zhongshan (113.35°E，22.53°N), Guangzhou (113.33°E，23.17°N) and Zhuhai (113.57°E，22.28°N). In January 2014, the PRD was frequently affected by cold front but there was no significant precipitation in the whole month. Figure 3 shows the daily mean 2m temperature, daily mean relative humidity, 24-hour surface pressure difference (e.g. difference of surface pressure at a certain time of a day and the day before; if the difference is positive in winter, possibly a cold front is affecting), and daily mean wind speed simulated by the WRF model as well as the observed data. It is found that the simulation results are close to the observation. The simulated 2m temperature in the three sites is slightly higher than the actual temperature in most of the time but the temporal variation fits the actual data well. The relative humidity is a little lower than the actual conditions. And the 24-hour surface pressure difference, which can represent the movement of the cold front, is basically equal to the actual conditions in the three sites. From the variations of temperature and surface pressure we can know that there were six cooling periods caused by cold front, namely 3-5, 8-9, 12-13, 18-19, 21-22 and 26-27 January. The simulated wind speed is stronger than the observed data in most days, especially in the days affected by cold front (e.g. 8, 18 and 21 January), but the tendency of the daily variation also fit the actual data well. Such results reveal that the WRF model gives an accurate simulation of the weather conditions in January 2014.

a)

Zhongshan             Guangzhou              Zhuhai

b)

c)

d)

Figure 3 Comparison of simulated a) daily mean 2m temperature, b) daily mean relative humidity, c) 24-hour surface pressure difference and d) daily mean wind speed with observed data (black lines for observation, and red lines for simulation).

3.2 Evaluation of CMAQ model results

Figure 4 is the comparison between the simulated PM2.5 concentration and visibility in Zhongshan in January 2014 and the observed data. Observed data used in CMAQ evaluation are from Zhongshan Protection Bureau and the hourly data are used. Currently there are four observation sites in Zhongshan, which are not far away from each other. So data from those four sites are averaged for model evaluation as well as the model output of grids where the four sites located. In general the model can accurately reproduce the temporal variation of PM2.5 concentration. However, the emission sources in the model can not completely illustrate the actual discharge of air pollutants, besides simulation errors exist in the results of the WRF model, making that during certain time periods the CMAQ simulation deviates from observation. When there is no cold front affects, wind speed simulated by the meteorological model is generally stronger than observation. At these times the simulated PM2.5 concentration are usually lower than the actual data, for instance on 5 and 23 January. When strong cold front moves southward, it transports a large amount of air pollutants to the downwind areas. If the cold front is strong enough, the pollutants can reach a longer distance. So if the simulated cold front is weaker than observation (simulated 24-hour surface pressure difference weaker than actual conditions), the model will weaken the transportation of extraneous pollutants, and further reduce the local simulated concentration, for instance on 12 January. Due to the fact that it was approaching the Chinese New Year from 28 to 30 January, the actual pollutant emissions were less than the usual. However, the emission sources used by the model fail to catch such variations, causing an obvious difference between the actual pollutant emission and the model simulation. So we can see that the simulated PM2.5 concentration is far higher than the observation during those three days. A PM2.5 concentration peak appears at about 08:00 on 31 January for the burning of firecrackers, which is usual during the Chinese New Year, but the model failed to capture this variation. On the aspect of visibility simulation, the model can reproduce the variation tendency well. However, as is mentioned above, when no cold front affects, the PM2.5 concentration from the model is usually lower than the observed data, causing higher simulated visibility than the actual one. But from 28 to 30 January the simulated visibility is lower than the observation because the simulated PM2.5 concentration is overestimated. The difference is mostly obvious when the observed or the simulated visibility reach their peak values. Since the simulated PM2.5 concentration is lower than the actual conditions in general, the simulated visibility is higher than the observation most of the time.

a)

b)

Figure 4. Comparison of simulated a) PM2.5 concentration and b) visibility with observed data (the discontinuities show observed missing values).

Table 3. Simulation errors of PM2.5 concentration and visibility.

|  | MB | ME | RMSE | MNB | NMB |
| --- | --- | --- | --- | --- | --- |
| PM2.5 Concertration | -13μg/m3 | 32μg/m3 | 29μg/m3 | -4% | -21% |
| Visibility | 0.8km | 4.9km | 5.6km | 8% | 20% |

Table 3 shows the simulation errors of PM2.5 concentration and visibility from CMAQ model. The calculation methods of statistical metrics, including average absolute bias (MB), average absolute error (ME), average root mean square error (RMSE), average normal deviation (MNB) and normal average deviation (NMB), are defined as follows:

MB=1ni=1nSimi-Obsi                   (1)

ME=1ni=1n|Simi-Obs(i)|                   (2)

RMSE=[1ni=1n(Simi-Obs(i))2]1/2            (3)

MNB=1ni=1n(Simi-Obs(i)Obs(i))                      (4)

NMB=i=1n(Simi-Obs(i))i=1nObs(i)                        (5)

where Sim and Obs represent simulated and observed values, respectively. It is found from Table 3 that the PM2.5 concentration from the model is lower than the observation, while the simulated visibility is a little overestimated. But in general the differences between simulation and observation are not evident, indicating that the simulation results are reliable.

4. Sensitivity analysis

4.1 Contributions of different regional emission sources

b)

a)

d)

c)

Figure 5. a) Spatial distribution of simulated monthly mean PM2.5 concentration; b) percentage contributions of emission sources of Zhongshan; c) percentage contributions of emission sources in other cities of PRD, and d) percentage contributions of emission sources outside Guangdong Province.

Figure 5a shows the spatial distribution of monthly mean PM2.5 concentration simulated by CMAQ. A relatively high PM2.5 polluted area is found in the middle of Foshan, where the average PM2.5 concentration exceeds 60μg/m3. PM2.5 concentration in the northwest of Zhongshan is 50-55μg/m3, while only 35-40μg/m3 in the southern part. It indicates that the PM2.5 concentration from northwestern to southeastern of Zhongshan is gradually reduced. Figure 5b is the contribution ratio of emission sources of local Zhongshan. The contribution ratio reaches 45% in the northwest of Zhongshan but reduces to 30-35% in the southern area. A part of areas of Jiangmen and Zhuhai, which locate in the downwind direction of Zhongshan, are also affected by the emission sources of Zhongshan. Figure 5c gives the contribution ratio of emission sources in other cities of PRD. Evidently the contribution ratio in the middle and southeast of Foshan exceeds 60%. Contribution ratio in southern Guangzhou, western Dongguan and Shenzhen, and eastern Jiangmen also reach 50% or more. These parts of emission sources contribute 35-45% PM2.5 in northern and 25-30% in southern Zhongshan. The emission sources outside Guangdong Province are very important to the entire areas of PRD (see Figure 5d). These parts of emission sources contribute relatively less nearby the southeast of Foshan, but the contribution ratio gradually increases in other parts of PRD. To Zhongshan, emission sources outside Guangdong contribute about 25% in the northern part but the contribution ratio increase to 35-40% in southern areas. From the discussion above it can be found that emission sources of local Zhongshan, other cities in PRD and emissions outside Guangdong Province are all very important to Zhongshan.

c)

b)

a)

Figure 6. Percentage contributions of the emission sources from different regions to the simulated PM2.5 in Zhongshan under a) no cold front, b) strong cold front and c) weak cold front case.

By selecting haze pollution processes generated under different weather conditions, the contribution ratio of each region to the PM2.5 concentration of Zhongshan can be figured out (Figure 6). When there is no cold front affects (24-hour surface pressure difference is negative), haze pollution is mostly caused by low wind speed and stable atmospheric stratification. Such weather condition is not beneficial for the long-distance transfer of air pollutants. So the contribution ratio of local emission sources of Zhongshan reaches 58%, and contribution ratio of emission sources from nearby cities in PRD and emissions outside Guangdong are 27% and 13%, respectively. When strong cold front (24-hour surface pressure difference is more than 3hPa) moves southwards, a large amount of non-local pollutants from regions beyond Guangdong Province are transported to PRD. It makes the contribution ratio of emissions outside Guangdong Province to hugely increase (45% in this case), followed by that of local emission sources of Zhongshan (28%) and other cities in PRD (23%). When under weak cold front (24-hour surface pressure difference is between 0hPa and 3hPa) weather conditions, the horizontal transportation of pollutants is enhanced to some extent, but such transportation is mainly dominated by short-distance one. So the contribution ratio of emission sources from other cities in PRD increase to 42%, followed by that of local emission sources of Zhongshan (34%). And the contribution ratio of emission sources outside Guangdong Province is also increased when compares to no cold front case, 21% at this time. Emission sources of other regions outside PRD in Guangdong Province do not contribute much to Zhongshan during January 2014.

It can be found from the results above that emission sources contributed differently to the haze pollution of Zhongshan under different weather conditions. Additionally, the simulated wind speed is usually higher than actual conditions. So the cross-region transportation of air pollutants may be overestimated to some extent, and the pollution in low wind speed cases may be weakened. Therefore, the model results may overestimate the contribution ratio of non-local emission sources, but underestimate the local one.

Table 4. Visibility growth of Zhongshan in sensitivity tests.

| Emissions be turned off | No cold front (%) | Weak cold front (%) | Strong cold front (%) |
| --- | --- | --- | --- |
| Zhongshan | 94 | 39 | 30 |
| Other cities in PRD | 30 | 76 | 27 |
| Outside Guangdong | 15 | 23 | 84 |

As showed in Table 4, it can be found that the local visibility increases after turning off emission sources of certain regions. When no cold front affects and the local emission sources of Zhongshan are turned off, the visibility is improved by up to 90%; when under weak cold front case and the emission sources of other cities in PRD are turned off, the visibility is improved by 76%; when a strong cold front affects and the emission sources outside Guangdong Province are turned off, the visibility is improved by 84%. Obviously the visibility does not increase in a linear way as the PM2.5 concentration decreases. The percentage of increment of visibility may be equal to the decrement of PM2.5 concentration when PM2.5 decreases slightly, but may be far higher than the decrement of PM2.5 concentration when PM2.5 decreases greatly.

d)

c)

a)

b)

Figure 7. Evaluation of simulated PM2.5 (μg/m3) and 10-m wind fields (m/s) at a) 08:00 LST (Local Standard Time); b) 17:00 LST; c) 23:00 LST on 8 January, and d) 17:00 LST on 9 January, 2014.

According to the analysis above, it can be found that emission sources outside Guangdong Province contribute most to the local PM2.5 concentration of Zhongshan when strong cold front affects PRD. Figure 7 shows the transportation process of PM2.5 during 8-9 January. It can be found from the figure that the most polluted area is located in Hunan Province on 8 January. As the cold front moves southwards, the aerosols are transported southwards and the PM2.5 concentration in northern Guangdong increases significantly. On the night of 8 January, a large amount of aerosols are transported to PRD and the western part of Guangdong by the cold front. By the afternoon of 9 January, the PM2.5 concentration in PRD is significantly decreased. Meanwhile, due to the diffusion, sedimentation and aging, PM2.5 concentration may decrease to some extent when pollutants are transported southwards. Similar transportation of pollutants is also available in other cold front cases. So air pollutants outside Guangdong Province may cause a large influence on the local haze pollution over Zhongshan in winter.

4.2 Impact of emission controls

This paper studies the impacts of emission controls of Zhongshan on local PM2.5 concentration and visibility. According to the analysis above, when under strong cold front weather conditions, the emission sources outside Guangdong contribute most to the PM2.5 concentration of Zhongshan. So in such days emission controls may not work perfectly. When there is no cold front, the contribution of local emission sources in Zhongshan increases greatly; therefore the emission control analysis in this study is performed under such weather conditions. In the emission control tests, the emission sources of Zhongshan are firstly turned off so as to observe the variation of PM2.5 concentration and visibility within 1-3 days after emission controls is taken. Then the industrial, residential and transportation sources of Zhongshan are respectively turned off to evaluate the impacts of different emission sources on PM2.5 concentration and visibility.

According to Table 5, when under no cold front weather conditions, PM2.5 concentration is decreased by 47%, 52% and 58% on the first, second and third day respectively after turning off the local emission sources of Zhongshan. The efficiency of emission controls apparently increases with the lapse of time. At the same time the local visibility improves by 57%, 63% and 90%. By the way, all results shown in Table 5 are obtained in ideal conditions of turning off emission sources for the whole Zhongshan. However, it is impossible to create such conditions when making actual emission controls. Therefore emission controls are suggested to be implemented 2-3 days before haze days to reduce PM2.5 by 50%.

Table 5. Percentage change of simulated PM2.5 concentration and visibility when local emissions are turned off in Zhongshan.

|  | PM2.5 concentration (%) | Visibility (%) |
| --- | --- | --- |
| 1st day | -47 | 57 |
| 2nd day | -52 | 63 |
| 3rd day | -58 | 90 |

Industrial, residential and transportation sources contribute significantly to local PM2.5 concentration. So those three parts of emission sources in Zhongshan are turned off to observe the variation of PM2.5 concentration and visibility during heavy polluted periods (average value of 5 hours before and after PM2.5 concentration reach the peak value is used here); such high pollution periods occur on the third day after emission controls start. It can be found from Table 6 that the emission sources which contribute most to PM2.5 concentration are industrial sources, and then residential and transportation sources. By turning off these three parts of emission sources, PM2.5 concentration is reduced by 24%, 22% and 15%, and the visibility is increased by 26%, 21% and 16%, respectively. Therefore, in conditions without an influence from a significant weather system, the government can consider emission controls from the perspectives of industrial, residential and transportation sources. For instance, restricting industrial production and reducing emissions of high pollution enterprises, using odd-and-even license plate regulations for motor vehicles, promoting a green lifestyle and reducing energy consumption so as to obtain an ideal effect.

Table 6. Percentage change of simulated PM2.5 concentration and visibility during heavy polluted periods after turning off industrial, residential and transportation sources of Zhongshan.

| Sources to be turned off | PM2.5 (%) | Visibility (%) |
| --- | --- | --- |
| Industrial sources | -24 | 26 |
| Residential sources | -22 | 21 |
| Transportation sources | -15 | 16 |

5. Conclusions

Scientific information on the contribution of emission sources is very important to the control of haze pollution but such studies are still not enough in Zhongshan so far. So the haze pollution over Zhongshan during January 2014 is simulated using the WRF-CMAQ model in this paper. The main conclusions, which can be used as a reference for haze control for government, are as follows:

The simulation results based on WRF-CMAQ model can reflect the evolution of the meteorological factors and the PM2.5 concentration of Zhongshan. PM2.5 concentration of Zhongshan gradually reduces from northwestern to southeastern. During January 2014, the contribution ratio of local emission sources reaches 45% in the northwest of Zhongshan but reduces to 30-35% in the southern area. Emission sources of other cities in RPD contribute 35-45% PM2.5 in northern Zhongshan but reduce to 25-30% in the southern area. The contribution ratio of emission sources outside Guangdong Province is 25% in the northern part but reaches 35-40% in southern Zhongshan.

The simulation tests show that the local emissions contribute most (58%) to the PM2.5 of Zhongshan under no cold front case. However when a strong cold front moves southward, a large amount of pollutants outside Guangdong Province are transported to the PRD region. That makes the contribution ratio of emissions outside Guangdong increases to 45%. When under weak cold front case, emissions of other cities in PRD contribute 42%, followed by the local emissions and emissions outside Guangdong.

Results of the emission control tests show that after turning off the whole local emission sources, PM2.5 concentration of Zhongshan is decreased by 47%, 52% and 58% on the first, second and third day. So emission controls are suggested to be implemented 2-3 days before haze days to reduce PM2.5 by 50%. The haze pollution of Zhongshan mostly come from industrial sources, followed by residential and transportation sources. After turning off industrial, residential and transportation sources, PM2.5 concentration is decreased by 24%, 22% and 15%, respectively.

Acknowledgments

This study is supported by the Key Projects in the National Science & Technology Pilar Program (2014BAC16B06), Project supported by the National Natural Science Foundation of China (41205123), Science & Technology Research Project of Guangdong Meteorological Service, China (2014B32), Special Research Project of Public Service Sectors (GYHY201306042). Also, the emission source inventory used in this paper is supplied by Center For Earth System Science from Tsinghua University, China. The authors would like to thank all people for their great work which helped to improve this manuscript.

References

[1] Y. C. Li, J. Z. Yu, S. H. H. Steven, Z. B. Yuan, A. K. H. Lau, and X. F. Huang, “Chemical characteristics of PM2.5 and organic aerosol source analysis during cold front episodes in Hong Kong, China”, Atmospheric Research, vol. 118, pp.41-51, 2012.

[2]X. X. Tie, D. Wu, and B. Guy, “Lung cancer mortality and exposure to atmospheric aerosol particles in Guangzhou, China”, Atmospheric Environment, vol. 43, no. 14, pp. 2375-2377, 2009.

[3]D. Wu, X. X. Tie, C. C. Li, Z. M. Ying, A. K. Lau, J. Huang, X. J. Deng, and X. Y. Bi, “An extremely low visibility event over the Guangzhou region: A case study”, Atmospheric Environment, vol. 39, no. 35, pp. 6568-6577, 2005.

[4]Q. Lu, J. Y. Zheng, S. Q. Ye, X. L. Shen, Z. B. Yuan, and S. S. Yin, “Emission trends and source characteristics of SO2, NOX, PM10 and VOCs in the Pearl River Delta region from 2000 to 2009”, Atmospheric Environment, vol.76, pp. 11-20, 2013.

[5]X. J. Deng, X. X. Tie, D. Wu, X. J. Zhou, X. Y. Bi, H. B. Tan, F. Li, and C. L. Jiang, “Long-term trend of visibility and its characterizations in the Pearl River Delta Region(PRD), China”,  Atmospheric Environment, vol. 42, no. 7, pp. 1424-1435, 2008.

[6]H. Huang, K. F. Ho, S. C. Lee, P. K. Tsang, S. S. H. Ho, C. W. Zou, S. C. Zou, J. J. Cao, and H. M. Xu, “Characteristics of carbonaceous aerosol in PM2.5: Pearl Delta River Region, China”, Atmospheric Research, vol. 104-105, pp. 227-236, 2012.

[7]Q. Fan, J. Lan, Y. M. Liu, X. M. Wang, P. Chan, S. J. Fan, Y. Y. Hong, Y. X. Liu, Y. J. Zeng, G. X. Liang, and Y. R. Feng. “Diagnostic analysis of the sulfate aerosol pollution in spring over Pearl River Delta, China”, Aerosol and Air Quality Research, vol. 15, pp. 46-57, 2015.

[8]P. F. Liu, C. S. Zhao, T. Gobel, E Hallbauer, N. Andreas, and L. Ran, “Hygroscopic properties of aerosol particles at high relative humidity and their diurnal variations in the North China Plain”, Atmospheric Chemistry and Physics, vol. 11, pp. 3479-3494, 2011.

[9]D. Wu, X. X. Tie, and X. J. Deng, “Chemical characterizations of soluble aerosols in Southern China”, Chemosphere, vol. 64, no. 5, pp. 749-757, 2006.

[10]J. H. Tan, S. J. Guo, Y. L. Ma, J. Duan, Y. Cheng, K. He, and F. Yang, “Characteristics of particulate PAHs during a typical haze episode in Guangzhou, China”, Atmospheric Research, vol. 102, pp. 91-98, 2011.

[11]D. Wu, J. T. Mao, X. J. Deng, X. X. Tie, Y. H. Zhang, L. M. Zeng, F. Li, H. B. Tan, and X. Y. Bi, “Black carbon aerosols and their radiative properties in the Pearl River Delta region”, Science in China Series D: Earth Sciences, vol. 52, no. 8, pp. 1152-1163, 2009.

[12]H. J. Xu, X. M. Wang, U. Poschl, S. L. Feng, D. Wu, Y. Ling, S. X. Li, W. Song, G. Y. Sheng, and J. M. Fu, “Genotoxicity of total and fractionated extractable organic matters in fine air particulate matter (PM2.5) in unban Guangzhou：comparison between haze and non-haze episode days”, Environmental Toxicology and Chemistry, vol. 27, no. 1, pp. 206-212, 2008.

[13]H. Yu, C. Wu, D. Wu, and J. Z. Yu, “Size distributions of elemental carbon and its contribution to light extinction in urban and rural locations in the Pearl River Delta, China”, Atmospheric Chemistry and Physics, vol. 10, no. 12, pp. 5107-5119, 2010.

[14]A. P. Kesarkar, M. Dalvi, A. Kaginalkar, and A. Ojha, “Coupling of the weather research and forecasting model with AERMOD for pollutant dispersion modeling: A case study for PM10 dispersion over Pune, India”, Atmospheric Environment, vol. 41,no. 9, pp. 1976-1988, 2007.

[15]P. Pongkiatkul, and N. T. K. Oanh, “Assessment of potential long-range transport of particulate air pollution using trajectory modeling and monitoring data”, Atmospheric Research, vol. 85, pp.3-17, 2007.

[16]W. P. Shan, Y. Q. Yin, H. X. Lu, and S. X. Liang, “A meteorological analysis of ozone episodes using HYSPLIT model and surface data”, Atmospheric Research, vol. 93, pp.767-776, 2009.

[17]W. W. Che, J. Y. Zheng, S. S. Wang, L. Zhong, and A. Lau, “Assessment of motor vehicle emission control policies using Model-3/CMAQ model for the Pearl River Delta region, China”, Atmospheric Environment, vol. 45, no. 9, pp. 1740-1751, 2011.

[18]R. Zhang, S. Golam, C. H. F Jimmy and K. H. L. Alexis, “Role of photoexcited nitrogen dioxide chemistry on ozone formation and emission control strategy over the Pearl River Delta, China”,  Atmospheric Research, vol. 132-133, pp. 332-344, 2013.

[19]Y. R. Feng, A. Y. Wang, D. Wu, and X. D. Xu, “The influence of tropical cyclone Melor on PM10 concentrations during an aerosol episode over the Pearl River Delta region of China: Numerical modeling versus observational analysis”, Atmospheric Environment, vol. 41, no. 21, pp. 4349-4365, 2007.

[20]F. Jiang, T. J. Wang, T. T. Wang, and M. Xie, “Numerical modeling of a continuous photochemical pollution episode in Hong Kong using WRF-chem”, Atmospheric Environment, vol. 42, no. 38, pp. 8717-8727, 2008.

[21]X. S. Wang, Y. H. Zhang, Y. T. Hu, W. zhou, L. M. Zeng, M. Hu, D. S. Cohan, and A. G. Russell, “Decoupled direct sensitivity analysis of regional ozone pollution over the Pearl River Delta during the PRIDE-PRD 2004campaign”, Atmospheric Environment, vol. 45, no. 28, pp. 4941-4949, 2011.

[22]L. J. Zhong, P. K. K. Louie, J. Y. Zheng, Z. B. Yuan, D. L. Yue, J. W. K. Ho, and A. K. H. Lau, “Science-policy interplay: Air quality management in the Pearl River Delta region and Hong Kong”, Atmospheric environment, vol. 76, pp. 3-10, 2013.

[23]J. Zheng, L. Zhang, W. Che, Z. Zheng, and S. Yin, “A highly resolved temporal and spatial air pollutant emission inventory for the Pearl River Delta, China and uncertainty assessment”, Atmospheric Environment, vol. 43, pp. 5112-5122, 2009.

[24]H. Liu, X. M. Wang, J. M. Pang, and K. B. He, “Feasibility and difficulties of China’s new air quality standard compliance: PRD case of PM2.5 and ozone from 2010 to 2025”, Atmospheric Chemistry and Physics, vol. 13, pp. 12013-12027, 2013.

[25]H. Christian, P. George, D. Wang. T. Alfreida, R. Shawn, P. Jonathan, and M. Rohit, “Annual application and evaluation of the online coupled WRF-CMAQ system over North America under AQMEII phase 2”, Atmospheric environment, vol. 115, pp. 683-694, 2015.

[26]X. S. Li, and R. Bernhard, “A WRF-CMAQ study on spring time vertical ozone structure in Southeast Texas”, Atmospheric Environment, vol. 97, pp. 363-385, 2014.

[27]W. C. Skamarock, J. B. Klemp, J. Dudhia, D. O. Gill, D. M. Barker, and M. G. Duda. “A description of the advanced research WRF version 3”. NCAR Tech. Note, NCAR/TN-475+STR, pp. 1-5,2008.

[28]F. S. Binkowski, and S. J. Roselle. “Models-3 community multiscale air quality (CMAQ) model aerosol component, 1. Model description”. Journal of Geophysical Research-Atmospheres, vol. 108, art. no. 4183, 2003.

[29]Q. Zhang, D. G. Streets, G. R, Carmichael,  K. B. He, H. Huo, A. Kannari, Z. Klimont, I. S. Park, S. Reddy, J. S. Fu, D. Chen, L. Duan, Y. Lei, L. T. Wang, and Z. L. Yao, “Asian emissions in 2006 for NASA INTEX-B mission”, Atmospheric Chemistry and Physics, vol. 9, pp. 5131-5153, 2009.

[30]Q. Zhang, G. N. Geng, S. W. Wang, A. Richter, and K. B. He, “Satellite remote sensing of changes in NO(x) emissions over China during 1996-2010”, Chinese Science Bulletin, vol.  57, pp. 2857-2864, 2012.

[31]K. B. He, “Multi-resolution Emission Inventory for China (MEIC): model framework and 1990-30 2010 anthropogenic emissions”, International Global Atmospheric Chemistry Conference, 2012.

[32]J. Y. Zheng, Z. Y. Zheng, Z. L. Wang, L. J. Zhong, and D. Wu, “Biogenic VOCs emission inventory and its temporal and spatial characteristics in the Pearl River Delta area”, China Environmental Science, vol.29, no. 4, pp. 345-350, 2009.

[33]Wang, L. T. Wang, J. M. Hao, He, K. B. He, S. X. Wang, J. H. Li, Q. Zhang, D. G. Streets, J. S. Fu, C. J. Jang, H. Takekawa, and S. Chatani , “A modeling study of coarse particulate matter pollution in Beijing: region source contributions and control implications for the 2008 Summer Olympics”, Journal of Air and Waste Management Association, vol. 58, pp. 1057-1069, 2008.

[34]J. Xing, Y. Zhang, S. X. Wang, X. H. Liu, S. H. Cheng, Q. Zhang, Y. S. Chen, D. G. Streets, C. Jang, J. M. Hao, and W. X. Wang, “Modeling study on the air quality impacts from emission reductions and a typical meteorological conditions during the 2008 Beijing Olympics”, Atmospheric Environment, vol. 45, pp. 1786-1798, 2011.