A modeling study of impact of emission control strategies on PM2.5 reductions in Zhongshan, China using WRF-CMAQ

Jianhua Mai1,2, Tao Deng2, Lingling Yu3, Xuejiao Deng2, Haobo Tan2, Shiqiang Wang4, Xiantong Liu2

1 Zhongshan Meteorological Service, Zhongshan 528400, China

2 Institute of Tropical and Marine Meteorology/Guangdong Provincial Key Laboratory of Regional Numerical Weather Prediction, Guangzhou, China Meteorological Administration, Guangzhou 510080, China

3 Guangdong Meteorological Observatory, Guangzhou 510080, China

4 Zhuhai Meteorological Service, Zhuhai 519000, China

Corresponding Author: Tao Deng

Institution: Institute of Tropical and Marine Meteorology/Guangdong Provincial Key Laboratory of Regional Numerical Weather Prediction, Guangzhou, China

Address: 6 Fujin Road, Yuexiu, Guangzhou, China

Tel: +86 13560376450

E-mail: tdeng@grmc.gov.cn

Abstract: A WRF-CMAQ modeling system is used to assess the impact of emission control strategies and weather conditions on haze pollution in Zhongshan, Guangdong Province, China. One month simulations for January 2014 are completed and evaluated with the surface observational data. The results show that the modeling system is able to reproduce the variations of meteorological fields and PM2.5 concentrations. Several sensitivity studies are completed to quantify the relative percentage contributions of local emissions versus regional emissions to the PM2.5 concentrations under different weather conditions. The results indicate that the contribution from local emission is higher than that of the emissions from outside (or regional transport) when no cold front appears in the Pearl River Delta (PRD) region (i.e., 58% contribution from local emission versus 42% contribution from the regional transport). The contribution of regional transport is increased to 76% when a strong cold front appears. Furthermore, the sensitivity study demonstrates that the emission reduction has accumulative effect and the effect is varied with weather conditions. Specifically, PM2.5 concentrations on the first, second and third day are reduced by 47%, 52%, and 58%, respectively, after the local emissions are turned off when there is no intrusion of cold front. Finally, a case study shows that industrial, residential and mobile emissions account for 24%, 22%, and 15% of the change of PM2.5, respectively, during a heavy haze pollution event in Zhongshan.

Keywords: CMAQ, Emission control, PM2.5, WRF, Zhongshan

1. Introduction

With rapid development of urbanization and industrialization, anthropogenic emissions have been increased significantly over the past several decades [1]. Haze has become a very serious air pollution problem in the Pearl River Delta region (PRD) and other regions in China [2]. High ambient levels of particulate matters with diameters less than 2.5 micrometer (PM2.5) can reduce visibility significantly, attenuate solar radiation, and pose harmful impact on human health [3-4].The observational data show that the number of haze episode days (the episode day is defined when daily mean PM2.5 concentration exceeds 75 µg/m3, the National Ambient Air Quality Standard for PM2.5 in China, NAAQSC for PM2.5) was over 100 in the PRD during the past decade [5]. The haze pollution has received a great concern from the governments and general public. Thus, understanding the relative contribution of local emissions and regional transport is extremely important to reduce PM2.5 concentrations.

Many research efforts have been devoted to investigate chemical composition characteristics of PM2.5, impact of weather systems and other meteorological fields on haze over the past several decades in the PRD region [6-13]. The monitoring data show that carbonaceous and inorganic secondary aerosols are the two major contributors to PM2.5 in the PRD [14]. Previous studies have pointed out that emissions outside of the PRD exert an important impact on PM2.5 in the PRD [15]. But they did not quantify the relative contributions. With the rapid development of computer technologies, numerical air quality models have become very useful tools for air pollution studies [16]. The Hybrid Single Particle Lagrangian Integrated Trajectory Model (HYSPLIT) was first used to track the sources of air masses that have important impact on haze pollution events in the PRD region [17-18]. Later, several more complicated numerical models were utilized to investigate the air pollution events [19-20]. For example, the Community Multi-scale Air Quality (CMAQ) model has been widely used to study pollution events in the PRD [21-25]. These studies have shown that haze pollution severity in the PRD is mainly controlled by anthropogenic emissions with strong spatial variation. They also pointed out that weather conditions are the key to trigger the haze pollution events [26]. However, very few studies have identified the relative contributions of local emissions and regional transport to PM2.5 concentrations under different weather conditions in the PRD region. Thus, accurate quantification of relative contributions of local emissions and regional transport to PM2.5 is essential to formulate effective emission control strategies.

Zhongshan suffers from similar severe haze pollutions to other cities in the PRD region. Study of impact of weather conditions and emissions control strategies on haze events is relatively lacking in Zhongshan as compared to other cities in the region [27]. 16 haze episode days were observed in January 2014. The offline coupling system of Weather Research and Forecast (WRF) with CMAQ is used to evaluate the impact of weather conditions and to quantify the relative contribution of local emission and regional transport to the haze events in Zhongshan. Sensitivity studies are designed to assess the contributions of different anthropogenic emissions such as industrial, residential, and mobile sources to the changes of PM2.5 during a heavy haze event. The study is aimed at providing scientific evidences for local policy-makers to develop more effective emission control measures.

2. Methods

2.1 Descriptions of models and simulation configurations

The WRF model (version 3.3.1) [28] is used to provide meteorological inputs to drive CMAQ (version 5.0.2). The National Center for Environmental Prediction (NCEP) Final Operational Global Analysis data with spatial resolution of 1°×1° and time interval of 6-hr are used to generate initial and boundary conditions for the WRF simulations. The WRF single-moment 6-class microphysics scheme (WSM6) and MM5 similarity surface layer scheme are used in the WRF runs. The Noah land-surface model and Yonsei University Boundary layer scheme (YSU) are also utilized in the WRF simulations. The CMAQ model is used to simulate different physical and chemical processes of gaseous pollutants and particulate matter (PM) compositions. The carbon bond (CB05) gas-phase chemical mechanism [29] and the AERO5 module [30] are utilized in the simulations. The detailed model configuration options are listed in Table 1.

Table 1. A summary of configurations used in the WRF-CMAQ simulations.

| Physics and Chemistry | Parameterization Schemes |
| --- | --- |
| Microphysics | WSM6 |
| Surface layer | MM5 similarity surface layer |
| Land-surface model | Noah |
| Boundary layer | YSU |
| Chemical gas-phase mechanism | CB05 |
| Aerosol module | AERO5 |

Figure 1a shows the modeling domains. Two-nested domains are used in WRF and CMAQ simulations with 31 vertical levels. The horizontal resolutions are 27-km and 9-km in coarse and inner domains, respectively. The exterior domain of CMAQ model covers the most regions of China and the interior domain covers the whole Guangdong Province. The locations of major cities in the PRD are given in Figure 1b. The simulations are conducted for the period from 08:00 LST (local standard time) on 25 Dec., 2013 to 08:00 LST on 01 Feb., 2014. The default boundary conditions are used for the CMAQ simulations in the CMAQ exterior domain. The first seven-day simulations are used as the spin-up runs to minimize the influence of default initial conditions.

b)

a)

Figure 1. a) Two-nested modeling domains for WRF (black solid boxes) and CMAQ ( blue dashed boxes) models; b) The inner CMAQ domain with three emission source regions and the major cities in the PRD region (Observational sites: Zhongshan(ZS), Guangzhou(GZ), Foshan(FS), Jiangmen(JM), Zhuhai(ZH), Dongguan(DG), and Shenzhen(SZ)).

Emissions are one of the most important factors to the air quality simulations. Many other studies have devoted to improve the emission inventory in China [31-32]. This study uses the Multi-resolution Emission Inventories for China (MEIC) version 2010, originally developed by Tsinghua University, China [33] and represents the latest emission inventory data in China. The emissions of MEIC are divided into five categories. They include the emissions from power plants, industry, agriculture, transportation and residential areas. The monthly mean emissions of gas pollutants such as SO2, NOX, CO and NH3, aerosol chemical compositions such as sulfate, nitrate, ammonium, black carbon, organic carbon, and other substances are used in the simulations. Given the resolution of 0.25°×0.25° for original emissions, the emission data are linearly interpolated into the inner domain and the curvature effect of the earth is taken into account during the interpolation.

Figure 2 shows the spatial distributions of annual mean emissions of NOx, SO2, CO and PM2.5 in the inner domain of CMAQ. It is found that major cities are the dominant sources of different emissions. For example, more than 500×106 mole of NOx, and 100×103kg of PM2.5 are emitted from Guangzhou, Foshan and Shenzhen each year. The temporal variations of the MEIC data are given by Zheng [34].

d)

b)

a)

c)

Figure 2. Annual emissions of a) NOx, b)SO2, c) CO (unit: ×106 mol/year/grid) and d) PM2.5 (unit: ×103 kg/year/grid) in the inner domain of CMAQ (cities of the PRD in the pictures: Zhongshan (ZS), Guangzhou (GZ), Foshan (FS), Jiangmen (JM), Zhuhai (ZH), Dongguan (DG) and Shenzhen (SZ)).

2.2 Contributions of emissions from different regions

To quantify the relative contributions of local and different regional emissions, the emissions are treated as four different cases: local emissions from Zhongshan, emissions from other cities in the PRD, emissions outside Guangdong Province, and emissions from other cities in Guangdong Province. Four scenarios are designed to evaluate the contributions of different emissions to PM2.5 concentrations by turning off individual emission. The details of different emissions are shown in Figure 1b and Table 2 (emissions from outside of Guangdong Province are not showed here). The first scenario, Test_ctr is the control simulation representing the benchmark with original emissions. The second scenario, Test_zs represents the case when the local emissions of Zhongshan are turned off. The third scenario, Test_prd is the case when the emissions of other cities in the PRD are turned off. This case is used to evaluate the influence of emissions from surrounding regions of Zhongshan. The last scenario, Test_gd is the case when the emissions from outside of Guangdong Province are turned off. The contribution of each region can be calculated by using the following formulas:

Cx=Cctr-Cx,0             (1)

Px=Cx/Cctr                     (2)

, where Cctr represents the PM2.5 concentration as the benchmark; Cx,0 is the PM2.5 concentration when the emissions of the region x are set to zero; Cx represents the difference of PM2.5 concentrations between emissions are turned on and off in the region; Px represents the contribution of emissions from region x. The similar method has been used by other air quality modeling studies of  2008 Beijing Olympic Game [35-36].

Table 2. A summary of experiment simulations.

| Experiment | Regions where Emission were turned off |
| --- | --- |
| Test_ctr | None |
| Test_zs | Zhongshan |
| Test_prd | PRD (excluding Zhongshan) |
| Test_gd | Outside of Guangdong Province |

3. Evaluation of meteorological and air quality model simulations

3.1 Evaluation of meteorological simulations

Meteorological inputs are important for air quality modeling study. Surface observational data at the sites of Zhongshan (113.35°E，22.53°N), Guangzhou (113.33°E，23.17°N) and Zhuhai (113.57°E，22.28°N) are used to verify the WRF simulations. Figure 3 shows the comparisons of WRF simulated daily mean 2m temperature, daily mean relative humidity, 24-hour surface pressure difference and daily mean wind speed with observational data in January 2014. Here the surface pressure difference denotes the one between the current day and one day before. The positive difference in winter indicates an intrusion of a cold front. It is seen from Figure 3 that the simulated 2-m temperature at the three sites is slightly higher than the observed temperature, but the temporal variation patterns are matched with the observations quite well. The relative humidity is a little lower than the observed value. The simulated 24-hour surface pressure difference shows excellent agreement with the observations at these three sites. It is found that six evident cooling periods were associated with the cold front activities, namely 3-5, 8-9, 12-13, 18-19, 21-22, and 26-27 January (the shaded areas in Figure 3). The wind speeds are over-predicted for most of the study period, especially for the days with the substantial impact from cold fronts (e.g. 8, 18, and 21 January). But the tendency of daily variations fits the actual value well. Overall, the simulations show that the WRF model is able to provide reasonable meteorological inputs for driving air quality modeling in January 2014.

b3)

b2)

b1)

a3)

a2)

a1)

d3)

d2)

d1)

c3)

c2)

c1)

Observation           Simulation

Figure 3 Comparison of simulated a) daily mean 2m temperature, b) daily mean relative humidity, c) 24-hour surface pressure difference and d) daily mean wind speed with the observations (1, 2 and 3 in the top-left corner of each panel represent Zhongshan, Guangzhou and Zhuhai station, respectively. The shaded areas represent the days with cold front intrusion).

3.2 Evaluation of CMAQ model results

Figure 4 shows the time series comparison of PM2.5 concentrations and visibility between the model simulations and observations in Zhongshan in January 2014. The hourly air quality observational data are provided by Zhongshan Environmental Protection Bureau and currently there are four observational sites in Zhongshan. Owing to the sites being close to each other, the averaged PM2.5 simulations are compared with the averaged observations at these four sites. As shown in Figure 4.a, overall the model is able to capture the general temporal variation patterns of PM2.5 concentrations at the observational sites. However, the model under-predicts most peak values of PM2.5 substantially and over-predicts the PM2.5 at some other time. The simulation biases are highly related to the uncertainties of emissions, the WRF and CMAQ simulations. The under-predictions of PM2.5 are usually associated with the over-prediction of wind speed when there is no intrusion of cold front. 5 January is an example for that. A large amount of air pollutants are transported to the downwind areas when a strong cold front moves southward. The pollutants can be transported over a longer distance when the cold front is strong enough. But the minor impact of extraneous transport on local PM2.5 concentrations is found if the simulated cold front is weaker than the observation (i.e., the simulated 24-hour surface pressure difference is less than observation). A typical example is illustrated on 12 January. Figure 4 also shows large impact of uncertainty of emissions on PM2.5 predictions. The anthropogenic emissions are much lower than the normal level during the 3-day period before Chinese New Year Eve (i.e., 28-30 January). The change has not included in the current emission inventory. As a result, the simulated PM2.5 concentrations are much higher than the observations during those three days. On the other hand, the model is not able to capture the PM2.5 peak value (236µg/m3) on the Chinese New Year Eve (i.e., 31 January). This is mainly because that the firework emissions are not included in the current emission inventory used in the simulations.

β=10+3×fRH×1.375×SO42-+1.29×NO3-+4×4.14×OC+                  10×EC+1×Soil+0.6×CM

Figure 4.b shows a comparison of simulated and observed visibility . It is noticed that  over-predictions of visibility are quite substantial during several days, especially for the days there is no intrusion of cold front. This is partly because the PM2.5 concentrations are usually under-predicted during these days (see 5 January as an example). It is reminded that humidity is another important factor to the change of visibility (see the formula in xxxx). The uncertainties of humidity simulation may cause the bias of visibility calculation based on the CMAQ simulations.

b)

a)

Observation          Simulation

Figure 4. Comparison of simulated a) PM2.5 concentrations and b) visibility with observations (The discontinuities are due to the missing observational data. The shaded areas denote the days with cold front intrusion)

Table 3. The evaluation statistics of CMAQ performance on PM2.5 (g/m3) and visibility (km).

|  | MB | ME | RMSE | MNB | NMB | COR |
| --- | --- | --- | --- | --- | --- | --- |
| PM2.5 | -13.4 | 32.1 | 29.2 | -4.0% | -21.4% | 0.43 |
| Visibility | 0.8 | 4.9 | 5.6 | 8.2% | 20.2% | 0.39 |

Several statistical metrics are used to evaluate the model performance. They include mean bias (MB), mean absolute error (MAE), root mean square error (RMSE), mean normalized bias (MNB), normalized mean bias (NMB) and correlation coefficient (COR). They are defined as follows:

MB=1ni=1nSimi-Obsi            (1)

MAE=1ni=1n|Simi-Obs(i)|           (2)

RMSE=[1ni=1n(Simi-Obs(i))2]1/2     (3)

MNB=1ni=1n(Simi-Obs(i)Obs(i))               (4)

NMB=i=1n(Simi-Obs(i))i=1nObs(i)                  (5)

COR=i=1n[(Simi-Sim)(Obsi-Obs)]i=1n(Simi-Sim)2×i=1n(Obsi-Obs)2    (6)

, where Sim and Obs represent simulated and observed values, respectively, and n is the number of the values. The evaluation statistics of the CMAQ performance on PM2.5 and visibility are shown in Table 3. It is found from that the simulated PM2.5 concentrations are lower than the observations and the simulated visibility is slightly over-predicted. And the correlation coefficient is 0.43 and 0.39 for PM2.5 concentrations and visibility, respectively. Overall the statistical calculations indicate that the simulation shows reasonable agreement with the observations.

4. Sensitivity analysis

4.1 Contributions of different regional emissions

b)

a)

d)

c)

Figure 5. a) Spatial distribution of simulated monthly mean PM2.5 concentrations; b) percentage contributions of local emissions in Zhongshan; c) percentage contributions of emissions from other cities in the PRD, and d) percentage contributions of emissions from outside of Guangdong Province.

Figure 5.a shows the spatial distribution of monthly mean simulated PM2.5 concentrations. An area with relatively high concentration of PM2.5 is found in the middle of Foshan, where the monthly mean PM2.5 concentrations exceed 60 μg/m3. PM2.5 concentrations in the northwest of Zhongshan are 50-55μg/m3, and only 35-40 μg/m3 is found in the southern part. The PM2.5 from northwestern to southeastern of Zhongshan is gradually reduced. Figure 5.b illustrates the percentage contribution of local emissions in Zhongshan. The contribution reaches 45% in the northwest of Zhongshan but reduces to 30-35% in the southern area. The downwind areas of Zhongshan including parts of Jiangmen and Zhuhai are also affected by the emissions from Zhongshan. Figure 5.c shows the percentage contributions of the emissions in other cities of the PRD (see Figure 1b). Evidently the contribution in the middle and southeast of Foshan exceeds 60%. Contributions in southern Guangzhou, western Dongguan and Shenzhen, and eastern Jiangmen are higher than 50%. These emissions contribute 35-45% of PM2.5 in northern and 25-30% in southern Zhongshan. In addition, the emissions outside Guangdong Province have important impact on the PRD region (see Figure 5.d). These emissions contribute relatively less over the southeast of Foshan, but the contribution gradually increases in other parts of the PRD. As to Zhongshan, the emissions outside Guangdong Province contribute about 25% in the northern part and increase to 35-40% in southern areas. Therefore, the emissions of local Zhongshan, other cities in the PRD, and outside Guangdong Province are all important to PM2.5 concentrations in Zhongshan.

4.2 Regional transport with different weather conditions

The weather conditions play an important role in regional air pollutant transport. Figure 6 shows the percentage contributions from different emission source regions under different weather conditions in Zhongshan. Haze pollution is mainly caused by low wind speeds and stable atmospheric boundary layer condition when there is no cold front intrusion (i.e., 24-hour surface pressure difference is negative). Meanwhile, such weather conditions are not beneficial to long-distance transport of air pollutants. So the contributions from local sources, other cities in the PRD, outside of Guangdong Province, and other emissions in Guangdong Province to the PM2.5 in Zhongshan are 58%, 27%, 13% and 2%, respectively. A large amount of non-local pollutants from outside of Guangdong Province are transported to the PRD region when a strong cold front (24-hour surface pressure difference is more than 3hPa) moves southwards. The contribution of the emissions from outside Guangdong Province is increased to 52% in this case, followed by local emissions of Zhongshan (24%), and other cities in the PRD (20%). The air pollutants are mainly controlled by short-distance transport when a weak cold front (24-hour surface pressure difference is about 0-3hPa) appears. The contribution of emissions from other cities in the PRD increase to 42%, followed by local emissions of Zhongshan (34%), and the emissions from outside Guangdong Province (21%)

It can be found from above that the emission contributions vary with different weather conditions. In addition, the simulated wind speed is usually higher than actual conditions. So the cross-region transport of air pollutants is overestimated to some extent. Therefore, the model results may overestimate the contribution of non-local emissions, but underestimate the local one.

c)

b)

a)

Figure 6. Percentage contributions of the emissions from different regions to the simulated PM2.5 in Zhongshan under a) no cold front, b) strong cold front, and c) weak cold front case.

According to the analysis presented above, it is found that emissions outside Guangdong Province contribute the most changes of local PM2.5 concentrations in Zhongshan when a strong cold front affects the PRD region. Figure 7 shows the impact of regional transport on PM2.5 in Zhongshan during 8-9 January. It is found that the most polluted areas were located in Hunan Province in the morning of 8 January. As the cold front moved southwards, the aerosol pollutants were transported southwards and caused large increase of PM2.5 concentrations in northern Guangdong Province. On the night of 8 January, a large amount of pollutants were transported to the PRD region and the western part of Guangdong Province. By the afternoon of 9 January, the PM2.5 concentrations in the PRD started to decrease. Meanwhile, PM2.5 concentrations were decreased due to the diffusion and sedimentation. Similar transport processes of pollutants were also observed for other cases with cold front intrusions. Thus regional transport of air pollutants from outside Guangdong Province poses a large impact on the local haze pollution in Zhongshan during winter.

d)

c)

a)

b)

Figure 7. The spatial distribution of simulated PM2.5 (μg/m3) and 10-m wind fields (m/s) at a) 08:00 LST (Local Standard Time); b) 17:00 LST; c) 23:00 LST on 8 January, and d) 17:00 LST on 9 January, 2014 (The names of the provinces in the pictures: Guangdong Province (GD), Guangxi Province (GX), Hunan Province (HN), Jiangxi Province (JX) and Fujian Province (FJ)).

4.3 Impact of local emission control on PM2.5 at different time periods

As discussed above, the emission control is the most effective to PM2.5 reduction when there is no cold front intrusion or the regional transport is not important. The effectives of local emission become much less when there is a strong cold front intrusion. In this section, we are focused on the impact of local emission control on the different stages. For this purpose, an additional simulation is completed for a single 3-day episode under no cold front condition. In this simulation, the whole emissions of Zhongshan area are turned off firstly, and then the industrial, residential and mobile emissions of local Zhongshan are turned off one by one to evaluate their respective impact on local PM2.5 concentrations.

Table 4 shows the changes of PM2.5 and visibility in Zhongshan on different days when local emissions are turned off and there is no cold front intrusion. It is clear that emission control has different impact on the different days. Specifically, PM2.5 concentrations are decreased by 47%, 52% and 58% on the first, second and third day respectively after the local emissions are turned off in Zhongshan. It is apparent that emission control has a clear phase effect. Similar phase effect is also observed for the changes of the local visibility which is increased by 57%, 63%, and 90% on the first, second, and third days, respectively. Therefore the large impact of local emission on PM2.5 tends to appear on the second and third day after the local emission are turned off. This suggests that the emission controls can be implemented on 2-3 days before a haze episode appears based on air quality forecasting.

Table 4. Percentage change of simulated PM2.5 concentration and visibility after local emissions are turned off in Zhongshan.

| The day after emissions are turned off | PM2.5 concentration (%) | Visibility (%) |
| --- | --- | --- |
| 1st day | -47 | 57 |
| 2nd day | -52 | 63 |
| 3rd day | -58 | 90 |

Industrial, residential, and mobile emissions are the three major emissions to local PM2.5 in Zhongshan given the limited impact of emissions from power plants. Now we evaluate the impact of the three emissions on PM2.5 in Zhongshan during a heavy haze pollution event. Here we evaluate their impact on PM2.5 during a peak period on the third day after the emission control is implemented. It is found from Table 5 that the industrial emission has the largest impact (-24%), the mobile emission has the smallest impact (-15%), and the residential emission stands between them when the three emissions are turned off individually. Meanwhile, the visibility is increased by 26%, 21% and 16%, respectively. Therefore, the government should consider the optimal implementation time of the emission control in order to reduce the frequency of occurrence and severity of heavy haze events.

Table 5. Percentage change of simulated PM2.5 concentration and visibility during heavy polluted periods after turning off industrial, residential and mobile emissions in Zhongshan.

| Emissions to be turned off | PM2.5 (%) | Visibility (%) |
| --- | --- | --- |
| Industrial emissions | -24 | 26 |
| Residential emissions | -22 | 21 |
| Mobile emissions | -15 | 16 |

5. Conclusions

An off-line coupling system of WRF-CMAQ is utilized to simulate PM2.5 and to quantify the relative contribution of local emission and regional transport to the change of PM2.5 in Zhongshan, China. A full month simulations are performed with the WRF-CMAQ modeling system and two-nested domains for January 2014. Several numerical experiments are conducted to investigate impact of the emissions from different regions and weather conditions on PM2.5 and visibility in Zhongshan. Both meteorological and PM2.5 simulations are evaluated with the surface observational data. A series of statistical parameters are used to evaluate the models’ performance. The results show that the WRF provides reasonable inputs for driving CMAQ. The CMAQ is able to capture the variation patterns of PM2.5 and visibility in Zhongshan.

The numerical sensitivity results show that the local emissions have the largest impact on PM2.5 in Zhongshan (58%) when there is no cold front intrusion. However, the role of emissions from outside Guangdong Province (or regional transport) tends to be more important (52%) when a strong cold front is extended to the PRD region. The emissions from other cities in the PRD become the most important one (42%) when a weak cold front appears.

A case study demonstrates that emission control has a clear phase accumulative effect on PM2.5 in Zhongshan under no cold front condition. PM2.5 concentrations of Zhongshan are decreased by 47%, 52% and 58% on the first, second, and third day, respectively, after the local emissions are turned off. This suggests that the emission control should be implemented on 2-3 days earlier than the day when a heavy haze event happens. Finally, the sensitivity study confirms that industrial, residential, and mobile emissions are the three major sources to PM2.5 in Zhongshan. Their relative contributions to PM2.5 are 24%, 22%, and 15% in Zhongshan during a peak period of the sensitivity simulations.

Acknowledgments

This study is supported by the Key Projects in the National Science & Technology Pilar Program (2014BAC16B06), Project supported by the National Natural Science Foundation of China (41205123), Science & Technology Research Project of Guangdong Meteorological Service, China (2014B32), Special Research Project of Public Service Sectors (GYHY201306042). Also, the emission source inventory used in this paper is supplied by Center For Earth System Science from Tsinghua University, China. The authors would like to thank all people for their great work which helped to improve this manuscript.

References

[1]T. Wang, X. L. Wei, A. J. Ding, C. N. Poon, K. S. Lam, Y. S. Li, L. Y. Chan, and M. Anson. “Increasing surface ozone concentrations in the background atmosphere of southern China, 1994-2007”, Atmospheric Chemistry and Physics Discussions, vol. 9, no. 16, pp. 10429-10455, 2009.

[2] Y. C. Li, J. Z. Yu, S. H. H. Steven, Z. B. Yuan, A. K. H. Lau, and X. F. Huang, “Chemical characteristics of PM2.5 and organic aerosol source analysis during cold front episodes in Hong Kong, China”, Atmospheric Research, vol. 118, pp. 41-51, 2012.

[3]X. X. Tie, D. Wu, and B. Guy, “Lung cancer mortality and exposure to atmospheric aerosol particles in Guangzhou, China”, Atmospheric Environment, vol. 43, no. 14, pp. 2375-2377, 2009.

[4]D. Wu, X. X. Tie, C. C. Li, Z. M. Ying, A. K. Lau, J. Huang, X. J. Deng, and X. Y. Bi, “An extremely low visibility event over the Guangzhou region: A case study”, Atmospheric Environment, vol. 39, no. 35, pp. 6568-6577, 2005.

[5]Q. Lu, J. Y. Zheng, S. Q. Ye, X. L. Shen, Z. B. Yuan, and S. S. Yin, “Emission trends and source characteristics of SO2, NOX, PM10 and VOCs in the Pearl River Delta region from 2000 to 2009”, Atmospheric Environment, vol. 76, pp. 11-20, 2013.

[6]X. J. Deng, X. X. Tie, D. Wu, X. J. Zhou, X. Y. Bi, H. B. Tan, F. Li, and C. L. Jiang, “Long-term trend of visibility and its characterizations in the Pearl River Delta Region (PRD), China”, Atmospheric Environment, vol. 42, no. 7, pp. 1424-1435, 2008.

[7]H. Huang, K. F. Ho, S. C. Lee, P. K. Tsang, S. S. H. Ho, C. W. Zou, S. C. Zou, J. J. Cao, and H. M. Xu, “Characteristics of carbonaceous aerosol in PM2.5: Pearl Delta River Region, China”, Atmospheric Research, vol. 104-105, pp. 227-236, 2012.

[8] P. F. Liu, C. S. Zhao, T. Gobel, E Hallbauer, N. Andreas, and L. Ran, “Hygroscopic properties of aerosol particles at high relative humidity and their diurnal variations in the North China Plain”, Atmospheric Chemistry and Physics, vol. 11, pp. 3479-3494, 2011.

[9]D. Wu, X. X. Tie, and X. J. Deng, “Chemical characterizations of soluble aerosols in Southern China”, Chemosphere, vol. 64, no. 5, pp. 749-757, 2006.

[10]J. H. Tan, S. J. Guo, Y. L. Ma, J. Duan, Y. Cheng, K. He, and F. Yang, “Characteristics of particulate PAHs during a typical haze episode in Guangzhou, China”, Atmospheric Research, vol. 102, pp. 91-98, 2011.

[11]D. Wu, J. T. Mao, X. J. Deng, X. X. Tie, Y. H. Zhang, L. M. Zeng, F. Li, H. B. Tan, and X. Y. Bi, “Black carbon aerosols and their radiative properties in the Pearl River Delta region”, Science in China Series D: Earth Sciences, vol. 52, no. 8, pp. 1152-1163, 2009.

[12]H. J. Xu, X. M. Wang, U. Poschl, S. L. Feng, D. Wu, Y. Ling, S. X. Li, W. Song, G. Y. Sheng, and J. M. Fu, “Genotoxicity of total and fractionated extractable organic matters in fine air particulate matter (PM2.5) in unban Guangzhou：comparison between haze and non-haze episode days”, Environmental Toxicology and Chemistry, vol. 27, no. 1, pp. 206-212, 2008.

[13]H. Yu, C. Wu, D. Wu, and J. Z. Yu, “Size distributions of elemental carbon and its contribution to light extinction in urban and rural locations in the Pearl River Delta, China”, Atmospheric Chemistry and Physics, vol. 10, no. 12, pp. 5107-5119, 2010.

[14]C. K. Chan, and X. H. Yao, “Air pollution in mega cities in China”, Atmospheric Environment, vol. 42, pp. 1-42, 2008.

[15]G. S. W. Hagler, M. H. Bergin, L. G. Salmon, J. Z. Yu, E. C. H. Wang, M. Zheng, L. M. Zeng, C. S. Kiang, Y. H. Zhang, A. K. H. Lau, and J. J. Schauer, “Source areas and chemical composition of fine particulate matter in the Pearl River Delta region of China”, Atmospheric Environment, vol. 40, 3802-3815.

[16]A. P. Kesarkar, M. Dalvi, A. Kaginalkar, and A. Ojha, “Coupling of the weather research and forecasting model with AERMOD for pollutant dispersion modeling: A case study for PM10 dispersion over Pune, India”, Atmospheric Environment, vol. 41, no. 9, pp. 1976-1988, 2007.

[17]P. Pongkiatkul, and N. T. K. Oanh, “Assessment of potential long-range transport of particulate air pollution using trajectory modeling and monitoring data”, Atmospheric Research, vol. 85, pp. 3-17, 2007.

[18]W. P. Shan, Y. Q. Yin, H. X. Lu, and S. X. Liang, “A meteorological analysis of ozone episodes using HYSPLIT model and surface data”, Atmospheric Research, vol. 93, pp. 767-776, 2009.

[19]J. M. Wan, L. M. Lin, C. Y. Chan, Z. S. Zhang, G Engling, X. M. Wang, I. N. Chan, and S. Y. Li, “Change of air quality and its impact on atmospheric visibility in central-western Pearl River Delta”, Environmental monitoring and assessment, vol. 172, pp. 339-351, 2011.

[20] Q. Fan, J. Lan, Y. M. Liu, X. M. Wang, P. Chan, S. J. Fan, Y. Y. Hong, Y. X. Liu, Y. J. Zeng, G. X. Liang, and Y. R. Feng, “Diagnostic analysis of the sulfate aerosol pollution in spring over Pearl River Delta, China”, Aerosol and Air Quality Research, vol. 15, pp. 46-57, 2015.

[21]W. W. Che, J. Y. Zheng, S. S. Wang, L. Zhong, and A. Lau, “Assessment of motor vehicle emission control policies using Model-3/CMAQ model for the Pearl River Delta region, China”, Atmospheric Environment, vol. 45, no. 9, pp. 1740-1751, 2011.

[22]R. Zhang, S. Golam, C. H. F Jimmy, and K. H. L. Alexis, “Role of photoexcited nitrogen dioxide chemistry on ozone formation and emission control strategy over the Pearl River Delta, China”, Atmospheric Research, vol. 132-133, pp. 332-344, 2013.

[23]Y. R. Feng, A. Y. Wang, D. Wu, and X. D. Xu, “The influence of tropical cyclone Melor on PM10 concentrations during an aerosol episode over the Pearl River Delta region of China: Numerical modeling versus observational analysis”, Atmospheric Environment, vol. 41, no. 21, pp. 4349-4365, 2007.

[24]F. Jiang, T. J. Wang, T. T. Wang, and M. Xie, “Numerical modeling of a continuous photochemical pollution episode in Hong Kong using WRF-Chem”, Atmospheric Environment, vol. 42, no. 38, pp. 8717-8727, 2008.

[25]X. S. Wang, Y. H. Zhang, Y. T. Hu, W. zhou, L. M. Zeng, M. Hu, D. S. Cohan, and A. G. Russell, “Decoupled direct sensitivity analysis of regional ozone pollution over the Pearl River Delta during the PRIDE-PRD 2004campaign”, Atmospheric Environment, vol. 45, no. 28, pp. 4941-4949, 2011.

[26]L. J. Zhong, P. K. K. Louie, J. Y. Zheng, Z. B. Yuan, D. L. Yue, J. W. K. Ho, and A. K. H. Lau, “Science-policy interplay: Air quality management in the Pearl River Delta region and Hong Kong”, Atmospheric environment, vol. 76, pp. 3-10, 2013.

[27]H. Liu, X. M. Wang, J. M. Pang, and K. B. He, “Feasibility and difficulties of China’s new air quality standard compliance: PRD case of PM2.5 and ozone from 2010 to 2025”, Atmospheric Chemistry and Physics, vol. 13, pp. 12013-12027, 2013.

[28]W. C. Skamarock, J. B. Klemp, J. Dudhia, D. O. Gill, D. M. Barker, and M. G. Duda. “A description of the advanced research WRF version 3”, NCAR Tech. Note, NCAR/TN-475+STR, pp. 1-5,2008.

[29] G. Sarwar, D. Luecken, G. Yarwood, G. Z. Whitten, and W. P. L. Carter, “Impact of an updated carbon bond mechanism on predictions from the CMAQ modeling system: preliminary assessment”, Journal of Applied Meteorology & Climatology, vol. 47, no. 1, pp. 3-14, 2008.

[30]W. T. Hutzell, D. J. Luechen, K. W. Appel, and W. P. L. Carter, “Interpreting predictions from the SAPRC07 mechanism based on regional and continental simulations”, Atmospheric Environment, vol. 46, pp. 417-429, 2012.

[31]Q. Zhang, D. G. Streets, G. R, Carmichael, K. B. He, H. Huo, A. Kannari, Z. Klimont, I. S. Park, S. Reddy, J. S. Fu, D. Chen, L. Duan, Y. Lei, L. T. Wang, and Z. L. Yao, “Asian emissions in 2006 for NASA INTEX-B mission”, Atmospheric Chemistry and Physics, vol. 9, pp. 5131-5153, 2009.

[32]Q. Zhang, G. N. Geng, S. W. Wang, A. Richter, and K. B. He, “Satellite remote sensing of changes in NO (x) emissions over China during 1996-2010”, Chinese Science Bulletin, vol.  57, pp. 2857-2864, 2012.

[33]K. B. He, “Multi-resolution Emission Inventory for China (MEIC): model framework and 1990-30 2010 anthropogenic emissions”, International Global Atmospheric Chemistry Conference, 2012.

[34]J. Y. Zheng, Z. Y. Zheng, Z. L. Wang, L. J. Zhong, and D. Wu, “Biogenic VOCs emission inventory and its temporal and spatial characteristics in the Pearl River Delta area”, China Environmental Science, vol. 29, no. 4, pp. 345-350, 2009.

[35]L. T. Wang, J. M. Hao, K. B. He, S. X. Wang, J. H. Li, Q. Zhang, D. G. Streets, J. S. Fu, C. J. Jang, H. Takekawa, and S. Chatani , “A modeling study of coarse particulate matter pollution in Beijing: region source contributions and control implications for the 2008 Summer Olympics”, Journal of Air and Waste Management Association, vol. 58, pp. 1057-1069, 2008.

[36]J. Xing, Y. Zhang, S. X. Wang, X. H. Liu, S. H. Cheng, Q. Zhang, Y. S. Chen, D. G. Streets, C. Jang, J. M. Hao, and W. X. Wang, “Modeling study on the air quality impacts from emission reductions and a typical meteorological conditions during the 2008 Beijing Olympics”, Atmospheric Environment, vol. 45, pp. 1786-1798, 2011.